.po 10

.he#Turbo DOS 1.42 8086 Update                T-Function 39#

.fo#Programmer's Guide         5-42-#         SET MODEM CONTROLS#

Turbo DOS 1.42 Update

Prepared by Advanced Digital Corporation

________________________________________________________________

#T-Function 39     Set Modem Controls#

Entry Arguments     |-------------------------------------------|

| Reg |        Description                  |

|-------------------------------------------|

|  CL   = 39                                |

|  DH   = channel, plus bit 7 if remote     |

|  DL   = modem control vector:             |

|         bit 7 set for request to send     |

|         bit 6 set for data terminal ready |

| (ADC)   bit 5 set for send break service  |

|         bit 4 reserved for future use     |

|         bit 3 reserved for future use     |

|         bit 2 reserved for future use     |

|         bit 1 reserved for future use     |

|         bit 0 reserved for future use     |

|___________________________________________|

Explanatio�        Th� Se� Mode� Control� functio� set� th� �

modem control signals in register DL on the

specified comm channel.

The setting of the modem control bits effects

the serial I/O device driver generated into

the operating system file (*.SYS).  Setting

of the modem control bits enables a service

in a serial I/O device driver.

Note: that the service is a function of the

eletrica� interfac� interconnectio� betwee� � �

seria� devic� an� � seria� devic� controller.

#*************

*  CAUTION  *

*************#

User� o� thi� functio� ar� requeste� t� �

revie� th� exac� interconnectio� fro� � �

seria� devic� t� � leve� converte� (PӠ NET� �

to a serial I/O controller. Further the user

should inspect the source for the serial I/O

driver to determine the precise response

elicited by the set of a modem control

vector bit.

##