File:  SLIPKT.DOC

SLIPKT Ver 0.56    Copyright 1999  -  [totally] ������ Software

Usage:

=====

C:\> SLIPKT [/? ][/d ][/Option Value ][/Option Value ]...

Options [default:min..max]  (any order,  / = DOS SwitchChar)

/b - Baud rate divisor (dec) [-:1..65535]

/c - COM port (AT) [1;1..4]

/d - Drop DTR on termination [no]

/f - FIFO trigger level [4:1,4,8,14]

/h - Hardware handshaking (check CTS) [no]

/i - IP address for RARP [none]

/p - I/O Port base address (hex) [3F8:0..3F8]

/q - IRQ [4:3..15(AT),2..7(XT)]

/s - Speed in bps (dec) [-:2..115200]

/v - Interrupt Vector [60h:60h..80h]

/? - Help [no]

Numbers specified as:  either  0xNN, NNh (hex)  or  NNN (dec).

Copyright, Licence, Warranty Disclaimer:

=======================================

See the FINE PRINT section at the end of this document.

Document Organization:

=====================

Synopsis.                       SLIPKT is ...

Options.                        Complete summary.

Getting Started.                Overview, requirements.

Detailed Comments.              Underview.

Related Software.               Helpful stuff.

Technical Summary.              Oh, is that right ?

Caveats.                        SLIPKT is *not* ...

Acknowledgements.               Without whom ...

FINE PRINT.                     Read this !

Synopsis:

========

SLIPKT is a DOS TSR which drives a PC serial port using

SLIP and provides an ethernet packet driver interface (FTP

Software, Class 1).  This allows TCP/IP-based application

software, which was designed to be used on an ethernet

connection, to be used with SLIP (Serial Line Internet

Protocol, RFC 1055).

SLIPKT was written from scratch in assembly language.

Its features include:

- Low memory overhead, about 2.6K.  This leaves a lot of

memory free for application programs, especially as

compared to PPP drivers.

- RARP emulation to simplify application configuration.

- Detects and uses FIFO buffers on 16550A or equivalent UARTs.

- Supports packet multiplexer PKTMUX' /b option.

Options:

=======

The options supported by SLIPKT must appear after the

DOS SwitchChar (represented here by '/' as is customary).

Most of the options are followed by one or more numerical

parameters.  Most numbers may be specified in either hex

or decimal form.  The hex forms 0xNN, 0NNh and NNh are

all recognized.  A brief summary of the options is

presented first, followed by a more detailed discussion.

Option        Meaning                 Example

------        -------                 -------

/b <div>      Baud rate divisor       /b 3

/c <com>      AT COM port             /c 3

/d            drop DTR on terminate   /d

(Hang up phone)

/f <trig>     receive FIFO trigger    /f 8

/h            Hardware handshaking    /h

/i <ip#>      IP address of PC        /i 192.168.17.25

/p <port>     Port's I/O base addr.   /p 3E8

(Must be in hex)

/q <irq>      port's hardware IRQ     /q 4

/s <speed>    DTE CONNECT Speed, bps  /s 38400

/v <vector>   software interrupt      /v 61h

Vector

/?            Help                    /?

WARNING:  When using the /c, /p or /q options, be sure you

are using the right values (do not guess)!  Using

the wrong values, including the default COM1 when

incorrect, can crash your machine and/or cause

unpredictable damage because the relevant port or

IRQ may already be in use by another device.

Baud Rate Divisor:  This specifies the divisor used to

set the line speed "baud rate" (DTE CONNECT speed).

If no /b (or /s) option is used, the default is to

retain the existing line speed.  Thus, modem users who

have set the line speed in their communications program

when dialing normally do not need this option.  The

parameter <div> to the /b option is *not* the baud rate

itself (e.g., 38400 bps), but rather a divisor in the

range 1..65535 which is used to select the baud rate.

The <div> parameter must be given in decimal (not hex).

This table shows some typical common examples:

Baud Rate      Divisor

57600           2

38400           3

19200           6

14400           8

9600          12

2400          48

300         384

The general relation is  <div> = 115200 / <speed>

(for a 1.8432 MHz crystal and 16 X Clock).  On

loading, both the divisor and implied speed are

displayed (under the same assumptions).  To set the

speed using the baud rate itself, rather than the

divisor, use the /s option.  Example:  For a divisor

of 8 (speed 14400 bps) use "/b 8".

DEFAULT:  existing speed

COM Port:  This specifies a "standard AT" COM port, i.e., a

(port base, IRQ) pair.  They are defined as follows:

COM1    COM2    COM3    COM4

port    3F8     2F8     3E8     2E8

irq       4       3       4       3

(SLIPKT does not attempt to determine COM port

addresses by inspecting the BIOS data area.)

Example:  For COM3 use "/c 3".  Must be in the

range 1..4.

DEFAULT:  COM1

Drop DTR:  This tells SLIPKT to drop the DTR (Data Terminal

Ready) signal to the modem when SLIPKT is terminated.

This can tell the modem to hang up an existing phone

connection.  Not all modems will hang up when DTR is

dropped, but they can usually be configured to do so.

Check your modem manual.

DEFAULT:  Do not drop DTR.

FIFO Trigger:  Specifies the trigger level of the receive

FIFO (first-in/first-out) buffer on 16550A UARTs.

This can be adjusted to try to optimize performance

under various circumstances.  Valid <trig> values

are 1, 4, 8 and 14 bytes.  Any other value will be

adjusted to a valid one.  See the Detailed Comments

section for more about the FIFO trigger.

DEFAULT:  4.

Hardware Handshaking:  Causes the driver to check that the

CTS (Clear To Send) bit of the modem status register

is set, before sending characters.

DEFAULT:  Disabled.

IP#:      The IP (Internet Protocol) address for your end of

of the SLIP connection.  If you are using a static

address, it will be assigned to you by your network

administrator.  If you use a dynamically assigned

address, it will vary from one connection to the next

(being assigned by the remote server software from a

pool of addresses).

The IP# option is used by SLIPKT solely to provide

responses to RARP requests.  Some applications use

RARP to find out the local IP address.  This is a

very convenient way to configure this parameter for

all such applications, in one step.  RARP is

supported if and only if the /i option is used.

Not using this option saves a small amount of memory.

DEFAULT:  none.

Port:     This can be used to specify the communication

port's I/O base address -- useful if your comm port

doesn't match one of the COM1..COM4 definitions.

This parameter *must* be given in hex, and so the

prefix "0x" and suffix "h" are not required.  For

example, /p 3E8, /p 0x3e8, /p3E8h are equivalent.

The valid range is 0..3F8.

DEFAULT:  3F8  (from COM1)

IRQ:      This specifies the hardware IRQ (interrupt

request priority level) used by the COM port --

again, useful when a standard COM port doesn't fit

your hardware configuration.  Example:  if your

modem uses IRQ 5, then use "/q 5".  Must be in the

range 3..15 for an AT class machine, or 2..7 for

a PC/XT.

DEFAULT:  4  (from COM1)

Speed:    This specifies the line speed "baud rate" directly.

If no /s (or /b) option is used, the default is to

retain the existing line speed (DTE CONNECT speed).

Thus, modem users who have set the line speed in their

communications program when dialing normally do not need

this option.  The parameter <speed> to the /s option is

the baud rate in bps, for example 38400.  The <speed>

parameter must be specified in decimal (not hex).  On

loading, the selected speed is displayed (it assumes

the usual 1.8432 MHz crystal and 16 X Clock).  To set

the speed using a baud divisor, use the /b option.

DEFAULT:  existing speed

Vector:   The software interrupt vector to be hooked by SLIPKT.

Applications use this vector to talk to the packet driver.

Must be in the range 60h ... 80h.  Typically, interrupts

60h through 67h are reserved for user purposes; it's

a good idea to choose a free one in this range.  Example:

To use vector 61h (= 97 decimal) use "/v 61h", "/v 0x61"

or "/v 97".

DEFAULT:  60h.

Help:     Get a help screen with "/?".

DEFAULT:  no help.

The space between an /Option and any parameter following

it is optional.  However you must leave a space before

each / delimiter.  The command line is case-insensitive.

Users of DOS 3.3 (truly `the best DOS ever') will be

pleased to know that a user-defined SwitchChar is supported.

Getting Started:

===============

This section is an overview.  Further details appear in

the next section.

In order to use this software, you need:

*    A SLIP account for Internet connection.

*    A modem supporting hardware flow control, or else a

direct link (or "NULL modem") connection to another

computer.

*    A PC with DOS version 2.0 or later.  (See also the

Caveats section below.)

*    If you are using a dial-up connection over a modem,

you'll need a communications, or dialer, program to

initiate, the connection.  It can also be used to close

the connection, or SLIPKT's /d option can hang up the

phone instead.

*    If you are using a direct serial link (rather than

a modem), you'll need to configure the serial port speed.

Normally you would use SLIPKT's /b or /s option to do

this, but you could instead use COMTOOL (see the next

section), or a full communications program.

*    TCP/IP application programs that use a Class 1 packet

driver.  They must be properly configured.  They must use

no protocols other than IP, ARP and RARP.

*    A  TERMIN.COM  program is helpful, to terminate/unload

SLIPKT (i.e. remove it from memory).  See below.

Once you have obtained all the necessary software, a

simple outline of one way to proceed is the following:

o   Configure your application program(s).  Depending on

the app, this may involve editing a configuration file,

running the program and going into a Setup menu, etc.

The main point to be made, at this time, is that when

configuring the `subnet mask' and `gateway ip address'

options for the app, you must use values that are

mutually consistent.  This means that:

<gateway_ip#> & <netmask> = <pc_ip#> & <netmask>

where  &  is bit-wise logical AND.  The gateway's

IP# should also differ from the PC's IP#.  If your

IP# is dynamically allocated, choose a netmask that

will encompass all possibilities, and choose a

gateway IP# obeying the above constraints.

In configuring the PC IP# for the app, either use

the correct value, or use "RARP" if the app

supports it and load SLIPKT with the correct

value using the /i flag.  For a dynamic address

session, the app IP# configuration step will have

to be done after logging in.

o   Set your modem and comm software to use hardware flow

control.

o   Either dialing `by hand' or with a script, get your

communications package to dial the phone and log in to

the SLIP account.  Then exit the communications program

without closing the connection.

o   For users of dynamic IP addressing, determine your

IP# for the session.  It is helpful to use a utility

(such as IPREAD -- see "Related Software" below) to

read it off the screen and store it in an environment

variable for later use.

o   Load SLIPKT.

SLIPKT  /i <ip#> [...]

If you don't use the /i flag to set the IP#, then

SLIPKT will not support RARP.  The /b or /s options

would normally be used to set the line speed only under

one of two circumstances:  (a) the speed set when

dialing by modem gets corrupted when exiting the

comm program; (b) you have a direct serial connection

without modem, and hence don't dial up.

o   Run your application(s).

o   To close the session, unload SLIPKT with

termin.com  (see below), e.g.,

TERMIN 0x60

and hang up the phone.

Detailed Comments:

=================

* Communications.

You can try whatever package you are familiar with, for

example MS-Kermit, Procomm, Telix, etc.  The advantage of

MS-Kermit is that it is both free, and (ver 3.13 and up)

supports TCP/IP in a way that is relatively easy to

configure.  Thus, telnet inside MS-Kermit can be one of

your applications.  To get started quickly, though, stick

to a package you know.

There are also a number of `dialer' programs available

on Internet which may do what is needed.  For example,

COMTOOL in:

ftp://mvmpc9.ciw.uni-karlsruhe.de/nos/sliplog/sliplog.zip

Another good choice is the CHAT0 dialer in Antonio Lopez

Molero's DOSPPP package (see CSLPKT.DOC for the URL).

This dialer is a port of Unix' CHAT with a number of nice

features added.

* Application Configuration.

Typically you must supply the application with some of

the configuration information discussed above, as well as

other things.  Remember to use `netmask' and `gateway'

values consistent with your IP#.  This ensures that the

app's TCP/IP stack will believe it can contact all

Internet sites.

If the app supports RARP, that may be used by the app

to determine the IP address.  This is particularly

convenient in the case of dynamic address assignment.

SLIPKT provides replies to RARP requests using the <ip#>

parameter you load it with.

See the file PKT_APPS.TXT for a modest list of some

popular applications, and suggestions on how to feed

them a dynamic IP address.

Other parameters such as various server addresses must

be obtained from your Internet provider or networking

guru.  TCP/IP parameters like max. segment size, max.

transmission unit, and receive window, may be required.

When using SLIPKT, the following maximum values for

these parameters can be used:

Max. Transmit Unit (MTU):               1006

Max. Segment Size (MSS, maxseg):         966

TCP Window:     small integer (2 or 4) * MSS

Using smaller values is OK too, and may give better

interactive (e.g., telnet key echo) performance when

concurrently transferring larger data segments (e.g.,

ftp).  See the discussion and formulae in CSLPKT.DOC.

Configuration varies from app to app; see the app's

documentation for further help.  See also PKT_APPS.TXT.

* About the FIFO Trigger Level.

The FIFO is a buffer on the 16550A UART chip (i.e., a

part of the hardware in some serial ports and modems).

In fact there are two FIFOs, one for transmit and one

for receive.  The latter, discussed here, is a 16-byte

buffer which helps prevent the loss of received

characters at high baud rates.  As characters arrive on

the serial line, they are first placed in the FIFO.  The

UART can be programmed to `trigger' a receive interrupt

when there are 1, 4, 8 or 14 characters in the FIFO.

This is the trigger level, <trig>.

If you choose a low value (say, 1) there will be lots

of interrupts, and their overhead is inefficient; if you

set it high (14) there will be fewer interrupts, but

maybe you increase the risk of losing characters when the

buffer gets full (there are then only 2 empty spots left

in the buffer).  You can tinker with different <trig>

values to see if one or another works better.  The

default of 4 should work pretty well in most cases.

Related Software:

================

+       A suitable terminate program is part of the Crynwr

Software packet driver collection.  Try:

ftp://boombox.micro.umn.edu:/pub/pc/packet-drivers/programs/termin.com

or for the whole collection look in:

ftp://ftp.simtel.net:/pub/simtelnet/msdos/pktdrvr/pktd11*.zip

+       The program IPREAD, from [totally] ������ Software,

is part of the IPCONFIG package:

ftp://ftp.simtel.net/pub/simtelnet/msdos/internet/ipcfg011.zip

It reads the dynamic address string reported when

logging in to a SLIP/PPP account and writes a batch

file that puts it into the environment.  Once there,

it can be used to configure applications.

+       Graham W Robinson's packet multiplexer, PKTMUX,

ftp://ftp.cc.rl.ac.uk/pub/pcsupp/network/pktmux/pktmux12.exe

is useful for running simultaneous applications over

a single packet driver in a multitasking environment.

Read its document carefully to understand its limitations.

Also, in trying this sort of thing, it's likely essential

to have a buffered UART (16550A).

+       Three other SLIP packet drivers you can try are:

(i)  Crynwr's EtherSLIP or SLIP8250, in

ftp://ftp.simtel.net:/pub/simtelnet/msdos/pktdrvr/pktd11*.zip

(ii)  Peter R. Tattam's SLIPPER and CSLIPPER,

ftp://jazz.trumpet.com.au/slipper/slippr15.zip

(iii)  University of Minnesota SLIP packet driver and dialer,

ftp://www.icdi.wvu.edu/INTERNET/UMSLIP24.ZIP

Technical Summary:

=================

SLIPKT conforms to revision 1.09 of the FTP Software

packet driver spec.  It is a Class 1 driver (Ethernet_II

or DEC/Intel/Xerox `Bluebook' Ethernet), of type 80

(SLIP-simulated ether).  Its functionality is 1 (basic,

no high-performance or extended functions).

SLIPKT emulates a local ethernet interface to the

application software, despite running over a SLIP link.

The machines deemed to be on this virtual ethernet are

determined by the netmask used to configure an

application.  SLIPKT does not care what this <netmask>

is, nor does it care what <gateway> you use.  The

application will use these values to determine when

and how to send ARP and RARP packets.  SLIPKT simply

assumes all such requests are legitimate.

In actual fact, the <gateway> IP number can usually be

chosen to be just about anything.  It need not even belong

to a real machine, provided that no application attempts to

send a datagram addressed *at the IP level* to the gateway.

What is important is that the <gateway>, <ip#> and <netmask>

parameters form a consistent set, so that applications

believe they can send and receive packets to and from the

gateway *at the ethernet level*.  However, to avoid

conflicts with your own IP#, it seems best to set <gateway>

to a suitable real machine.

SLIPKT allows applications to register any protocols

having 2-byte ethertypes.  *However*, all protocols

except for IP, ARP and RARP are ignored.  An application

making essential use of other protocols will not function.

For example, you cannot directly receive or transmit IPX

packets.  This is a design limitation of SLIP.  Because of

this 2-byte restriction, some older versions of TERMIN.COM

will not work.  If you must use other protocols, consider

PPP instead of SLIP.

When appropriate, SLIPKT internally generates replies

to ARP and RARP packets sent by the application, while

all IP packets are sent out over the link.  Packets

received from the SLIP link should only be IP.  Incoming

datagrams are passed to the app's receiver, if their

actual size matches the size found in the IP header.  IP

datagrams larger than 1006 bytes are truncated, and so

will not generally be passed to the receiver.

The application's receive handler for ARP and RARP

packets must be capable of handling replies before the

corresponding send routine has terminated.  ARP requests

for the local <ip#> are ignored, to placate applications

checking for a duplicate address.  RARP replies are

provided only for the local (fictitious) hardware

address, as the local IP# (if specified by /i) is the

only one SLIPKT knows about.

A strict MRU (Maximum Receive Unit) of 1006 byte IP

datagrams is enforced internally, as noted above.  This

is necessary to avoid receive buffer overrun.  While it

might be nice to enforce an MTU similarly, this isn't done:

there are certain applications (e.g., NuPOP, Minuet) which

transmit packets as large as they please (regardless of the

MTU configured in their Setup/Network menu).  Enforcing an

MTU of 1006 would render such programs unable to do ftp

puts.  This looseness does little harm, but users of such

programs (or users who choose large MTUs in other apps)

should be aware that not all C/SLIP servers support this.

SLIPKT should work with unbuffered UARTs (8250, 16450).

It detects a 16550A or equivalent buffered UART on loading,

and enables and uses its FIFOs.  The receive trigger can

be changed from the default value using the /f flag.  If

no FIFO is found, this flag has no effect.

On loading, SLIPKT configures the serial port to 8N1

data/parity/stop bits.

SLIPKT supports the /b option of PKTMUX ver 1.2i.  This

option is used to increase efficiency by reducing the

amount of packet copying during reception.

Caveats:

=======

-   SLIPKT is not a "SLIP class" packet driver (Class 6

of revision 1.11 of the FTP Software spec.)

-   The emulation of ethernet over a SLIP link inevitably has

shortcomings, some of which are mentioned in the above

section.  Clearly, ethernet-speed connectivity over

dialup lines will also not be achieved with current modem

technology.  If the application is sensitive to such

timing matters, problems can arise.

-   SLIPKT has not been tested with MicroSoft Windows.

-   SLIPKT has not been extensively tested with PKTMUX.

You will almost certainly need a buffered UART for this

to have any chance of working.  Multiplexing with PKTMUX

is a somewhat aggressive strategy, so be prepared for at

least occasional system lockups.  Read the PKTMUX document

carefully and back up all data first.

-   SLIPKT requires hardware flow control to be enabled.

It will not escape XON and XOFF flow control characters.

-   SLIPKT provides no BOOTP emulation.

-   SLIPKT does not support the sharing of a common

interrupt for COM2 through COM8 on PS/2 computers.

-   SLIPKT will not share an IRQ with another device.

Make sure that all devices to be used while SLIPKT is

loaded use a different IRQ.

-   SLIPKT allows IRQs in the range 8..15, and disallows

use of IRQ2, if it determines that two PICs (Progammable

Interrupt Controllers) are present (essentially, that

the machine is AT class).  It does not, however, check

for the existence of a 2nd PIC by hardware I/O means.

Because this determination may potentially be fallible,

please ensure that the IRQ you specify is correct (i.e.,

being used only by your serial port).  In particular,

IRQs 8..15 should not be used on a PC/XT.  On an AT,

IRQ2 should not be used; the corresponding bus data

line is connected to and identified as IRQ9.

Acknowledgements:

================

(1)    The FTP Software packet driver specification was developed

and is Copyright by:

FTP Software, Inc.

2 High Street

North Andover, MA 01845

(508) 685-4000

DESQview is a trademark of:

Quarterdeck Office Systems

150 Pico Boulevard

Santa Monica, CA 90405

(310) 392-9851

Microsoft Windows is a registered trademark of Microsoft Corp.

PS/2 is a trademark of IBM Corp.

(2)    NOTE:  SLIPKT is archived using Info-ZIP's compression

utility.  Info-ZIP's software (Zip, UnZip and related

utilities) is free and can be obtained as source code or

executables from various bulletin board services and

anonymous-ftp sites, including CompuServe's IBMPRO forum

and  ftp://ftp.uu.net:/pub/archiving/zip/* .

(3)    Thanks to the beta and pre-beta testers, for their time

and helpful comments.  Their feedback and tireless

cooperation is appreciated and most valuable.

(4)    Special thanks to Joe Doupnik for sage advice about

serial packet transmission, and to Graham Robinson for

communicating his insights into PKTMUX.  (Any bugs are,

of course, not due to Joe or Graham.)

(5)    Thanks also to Toni Molero and Jeff Hayes for many

interesting and helpful discussions.  Jeff also included a

link to the beta CSLPKT web page on his DOS Internet Page.

FINE PRINT:

==========

All products mentioned in this documentation which are

patented, copyrighted or are trademarks are the property

of their respective owners.

SLIPKT is Copyright 1999 by Frank Molzahn.

All rights reserved.

SLIPKT is smiley-ware.  If you think this software is

worth using (or maybe you just like the DOC), why not

send the author your smiley?

SLIPKT is part of the CSLPKT05 package.

SLIPKT may be freely distributed provided all the files in

the package are present and unmodified.  This specifically

includes the files

BTSA.TXT

SLIPKT.COM

SLIPKT.DOC

CSLPKT.COM

CSLPKT.DOC

PKT_APPS.TXT

README.TXT

This copyright, licence and warranty disclaimer notice

applies to the entire CSLPKT05 package.  If you are unsure

whether or not your package is intact, download the current

version from Simtel.Net.

SLIPKT IS PROVIDED "AS IS" WITHOUT WARRANTY OF ANY KIND,

either expressed or implied, including, but not limited

to, the implied warranties of merchantability and fitness

for a particular purpose.  The entire risk as to the

quality and performance of the product is with the user.

This documentation explicitly states that SLIPKT will not

work properly in some circumstances; the user assumes the

cost of all necessary servicing, repair or correction.

In no event will Frank Molzahn be liable for damages,

including any general, special, incidental or consequential

damages arising out of the use of, or inability to use,

SLIPKT (including but not limited to loss of data or data

being rendered inaccurate or losses sustained by the user

or third parties or a failure of the product to operate

with any other programs), even if the author has been

advised of the possibility of such damages.

These copyright, license, and disclaimer notices must be

included with all copies of SLIPKT.

You may use SLIPKT only if you have read, understood and

accepted all of the above conditions.

Please contact me to report bugs or errors in the document

(even minor ones).  If you have constructive suggestions for

improving SLIPKT I'd be glad to hear from you.

[totally] ������ Software

is:

Frank Molzahn                  molzahn@cc.umanitoba.ca [preferred]

Winnipeg, Canada               molzahn@wotan.physics.umanitoba.ca

http://home.cc.umanitoba.ca/~molzahn/software.html