zavt	- enhanced MS-DOS ansi/avatar console driver

SYNOPSIS

Include in \config.sys the line

device=zavt.sys

DESCRIPTION

Zavt.sys is a console driver which understands ANSI control sequences

as well as Avatar level 0. It has several advantages over most other

drivers.

1. It supports any display mode the BIOS can set up (see below)

2. It can be addressed by two different names so the default

output can be left in raw mode (see below)

3. The blasted bell is now 1/9 second

4. Any DOS console entry may make use of a full-screen editor

One trivial advantage is the allowing of backspaces across the left

margin. I don't know what problems this may create, but it is nice

for the command line.

COMPATABILITY

-------------

Zavt gets all its information about the console from the BIOS data

area - nothing is assumed other than that modes 0-3 and 7 are text

modes. Other drivers assume your screen is 25 lines long, and that

the video card is always left on page 0. This is a safe assumption

most of the time, but bad practice. AutoCAD 2.5 for example is more

efficient in switching between graphics and text when you configure

it to write text on page one and use page 0 for its graphics, but

will not give a usable text page with other drivers. Nansi has code

in it to switch an EGA to 43 line mode, or an AT&T display to 50 line

mode, but if you switch back by any means other than Nansi's own

facilities, Nansi continues to assume 43 lines. Zavt responds to

changes in display size very well. If you switch to a mode with more

lines on screen (not by means of the driver - system specifics I

leave to other software), Zavt will use them immediately. If you

switch to a mode with fewer lines and the cursor is off the new

screen, the next scroll will bring it along with the screen region

around it into view. The best way to get a feel for this if you have

an EGA is to play with the xxLINE.COM programs included. The names

are for EGA, and will work for VGA, but will give you more lines than

advertised.

DEVICE DRIVERS

--------------

When you, for example, type

C:> type foo.txt

COMMAND.COM opens the file foo.txt, reads it, and writes it to

the console driver, which puts it up on the screen. DOS has

two ways of writing to the console driver:

raw/binary mode: where the device driver is called once for

a group of characters letting the device driver

handle them all at once, or

cooked/ASCII mode: where DOS calls the device driver once for each

character to be sent, expanding any tabs.

Raw mode is much faster as the device driver is called fewer

times requiring less overhead all together. However, you lose

the ability to abort in the middle of output. If you TYPE a large

file, COMMAND.COM sends it to the console driver 32K at a time, so

if you Ctrl-Break, you'll have to wait until the first 32K have been

displayed. DOS also no longer expands tabs, but this is easily

handled by the device driver. Run RAW.COM then type

C:> dir /w

and rather than tabs between filenames, the tab character is displayed.

Run COOKED.COM before continuing.

When DOS wants input from the console (such as when reading input

for a filter such as MORE), raw and cooked mode also apply. In

cooked mode, DOS will echo each character you enter, and interpret

the cursor keys and f-keys allowing you to edit what you enter. DOS

will also end entry when you hit CR, returning what you typed up

to that point, even if more characters were requested. To demonstrate

this, type

C:> more

and type some gibberish. DOS is requesting 4096 characters, but as soon

as you type <enter>, entry ends and more prints out what you typed.

Press f6 and enter to end.

When the console is in raw mode however, DOS does not echo

characters, check for ^C, ^S, or ^P, and asks of the driver however

many characters the calling program wants. The driver has no choice

but to oblige, and without feedback, the computer seems frozen. Run

RAW.COM included in this archive, then try MORE. Unless you type 4096

characters, with a ^Z included to flag end of file, the computer is

for all intents and purposes crashed.

So while setting the console to raw mode lets you display text

*MUCH* faster, you have a potential trap if you run into software

which requests data from the default input. The solution would seem

to be to write in raw mode, and read on cooked mode, but DOS will

speak to a device driver in only one way at a time. The solution?

Use a second device driver. The console is normally addressed as

CON, but Zavt has a secondary handler which allows it to be addressed

as KEYB as well. The problem is how to change the default input to

KEYB. CTTY will change both the default input, but it will change

the default output and error output as well. In DOS versions up to

3.3, CTTY has a bug which will allow you to

C:> CTTY <KEYB

DOS will say 'Invalid Device' but otherwise acts as expected. DOS

4.0 users will have to live with the risk (it's not that great).

Once the default input is set to the alternate handler, you can

go ahead and set the default output (and with it, the error output)

to raw mode.

ANSI (taken from documentation for NANSI 2.4)

----

While putting text up on the screen, zavt.sys keeps a lookout for

the escape character (chr(27), known as ESC); this character signals

the start of a terminal control sequence.

Terminal control sequences follow the format

ESC [ param; param; ...; param cmd

where

ESC	is the escape character chr$(27).

[	is the left bracket character.

param	is an ASCII decimal number, or a string in quotes.

cmd	is a case-specific letter identifying the command.

Usually, zero, one, or two parameters are given.  If parameters

are omitted, they usually default to 1. Spaces are not allowed

between parameters.

For example, both ESC[1;1H and ESC[H send the cursor to the home

position (1,1), which is the upper left.

Either single or double quotes may be used to quote a string.

Each character inside a quoted string is equivalent to one numeric

parameter.  Quoted strings are normally used only for the Keyboard

Key Reassignment command.

ESCAPE.COM from PC-Magazine is included as an easy way to send

these control sequences, sending the ESC and [ followed immediately

by whatever is given to it on the command line.

Control Sequences

The following table lists the sequences understood by nansi.sys.

Differences between nansi.sys and the standard ansi.sys are marked

with a vertical bar (|).

Cursor Positioning

Short	Long name		Format		Notes

CUP	cursor position		ESC[y;xH	Sets cursor position.

HVP	cursor position		ESC[y;xf	Same as CUP; not recommended.

CUU	cursor up		ESC[nA		n = # of lines to move

CUD	cursor down		ESC[nB

CUF	cursor forward		ESC[nC		n = # of columns to move

CUB	cursor backward		ESC[nD

DSR	Device Status, Report!	ESC[6n		Find out cursor position.

CPR	Cursor Position report	ESC[y;xR<CR>	Response to DSR, as if typed.

SCP	Save Cursor Position	ESC[s		Not nestable.

RCP	Restore Cursor Position ESC[u

Editing

EID	Erase in Display	ESC[J 	Clears screen to cursor

EID	Erase in Display	ESC[1J	Clears screen from cursor.

EID	Erase in Display	ESC[2J	Clears screen.

EIL	Erase in Line		ESC[K	Clears to end of line.

EIL	Erase in Line		ESC[1K	Clears line up to cursor.

EIL	Erase in Line		ESC[2K	Clears entire line.

IL	Insert Lines		ESC[nL	Inserts n blank lines at cursor line.

DL	Delete Lines		ESC[nM	Deletes n lines, including cursor line.

IC	Insert Characters	ESC[n@	Inserts n blank chars at cursor.

DC	Delete Characters	ESC[nP	Deletes n chars, including cursor char.

Mode-Setting

SGR	Set Graphics Rendition	ESC[n;n;...nm	See character attribute table.

SM	Set Mode		ESC[=nh		See screen mode table.

RM	Reset Mode		ESC[=nl		See screen mode table.

IBMKKR	Keyboard Key Reass.	ESC["string"p

The first char of the string gives the key to redefine; the rest

of the string is the key's new value.

To specify unprintable chars, give the ASCII value of the char

outside of quotes, as a normal parameter.

IBM function keys are two byte strings; see the IBM Basic manual.

For instance, ESC[0;59;"dir a:";13;p redefines function key 1 to

have the value "dir a:" followed by the ENTER key.

If no parameters given, all keys are reset to their default values.

Character Attributes

The Set Graphics Rendition command is used to select foreground

and background colors or attributes.

When you use multiple parameters, they are executed in sequence, and

the effects are cumulative.

Attrib code		Value

0		All attributes off (normal white on black)

1		Bold

2		Bold off

4		Underline

5		Blink

7		Reverse Video

8		Invisible (but why?)

30-37		foregnd blk/red/grn/yel/blu/magenta/cyan/white

40-47		background (no effect in graphics modes)

Screen Modes

The Set Mode and Reset Mode sequences both accept one parameter,

with the following values:

Mode Code	Value (same for Set and for Reset)

0	text 40x25 Black & White

1	text 40x25 Color

2	text 80x25 Black & White

3	text 80x25 Color

4	bitmap 320x200 4 bits/pixel

5	bitmap 320x200 1 bit/pixel

6	bitmap 640x200 1 bit/pixel

13	EGA graphics 320x200 4 bits/pixel

14	EGA graphics 640x200 4 bits/pixel

15	EGA [ IBM Monochrome display 640x350 2 bits/pixel?]

16	EGA graphics 640x350 4 bits/pixel [with full 256K]

Mode Code	Set			Reset

7      *Wrap at end of line 	No wrap at EOL

45     *Graphics Cursor		No simulated graphics cursor

Note: Modes other than the two set codes are BIOS display modes

and modes not listed which may be supported by new cards

(VGA for example) can also be selected by these functions.

[ * = default ]

Little Dead Blobs

If you are using a graphics mode, and the cursor leaves little

dead blobs behind when it moves, you are probably using a program

which bypasses DOS (naughty, naughty).  It might help to tell

Nansi to turn off its graphics cursor by sending the string

ESC [ = 45 l

(The last character in that string is a lowercase "L".)

AVATAR

------

While putting text up on the screen, Zavt.sys also looks out for

^V, ^Y, ^L and ^P. The following is taken from documentation for

OANSI.

ALIASING

In an Avatar sequence, the DLE character is "escaped."

That's a scheme to let you use characters normally not available

to an editor.

DLE    Hex=10 Decimal=16

Here are some examples of character aliases:

Hex      Decimal    Real character

----------------------------------

10 83    16 131      3   (^c)

10 9a    16 154     1a   (^z)

10 90    16 144     10   (^p)

10 10    16  16     10   (^p)

When an Avatar terminal runs into a DLE character, it grabs the next

available byte and strips off the high-order bit.

CONTROL CODES

Command           Description

-----------------------------------------------

^v ^a <a>	attribute

The "<a>" is a single

byte.  The value of this

attribute byte corresponds

to the video attributes

used by IBM on their CGA

monitors.  (Intense video

is supported, black-on-black

is not.)

^v ^b		blinking on

blinking is turned off

by the next "^v^a"

command

^v ^c		move cursor up a line

^v ^d		move cursor down a line

^v ^e		move cursor left a line

^v ^f		move cursor right a line

^v ^g		clear to end of line

^v ^h <r> <c>	set cursor position

These are 1-based numbers

with the "Y" (row) coordinate

first.  The upper left of the

display is "1,1."  The start

of the second line would be

"2,1."

^l		set current attribute to cyan

on black, home cursor and

clear screen

Avatar is a binary based protocol while ANSI is ASCII based.

As a result, control codes which may have other meanins may

end up part of an Avatar control sequnce. This is not a problem

except when something tries to interpret a control code without

realizing it is to be preserved. The only situation where this

occurs that is of any concern is with DOS. If the console is in

cooked mode, any ^i will become some number of spaces as DOS

considers it to be a tab. This will corrupt any Avatar code with

a ^i in it. The solution is to set the console to raw mode, which

is the reccommended setup for speed as well.

Full-Screen input

-----------------

Code for the full-screen editor was mostly taken from PC-Magazine's

DOS-EDIT and thus the mechanics are virtually identical. Being part

of the console handler rather than a TSR however allows the editor

to be used for any DOS input rather than only the command line.

For those not familiar, pressing cursor-up on the command line

with DOS-EDIT loaded would move the cursor up one line and put

you into the full-screen editor. From there, the special keys are

Escape		leave the fullscreen routine

Cursor keys	move the cursor accordingly

Home		move the cursor to the left margin

End		leave the full-screen routine, putting what is

on the screen under and following the cursor

into a buffer to be read as keyboard input

PgUp		move the cursor to its original column

PgDn		delete all characters on and following the cursor

Insert		toggle insert/delete mode

Delete		delete character to left of cursor and move text

under and to the right of the cursor one space left

Enter		same as End but adds a CR to the end of the data

The alpha-numeric keys print to screen while control codes

may or may not depending on whether or not they have other purposes.

Alt-<num> input always prints however, so it is possible to queue

a sequence of strings by putting typing them on one line and typing

13 on the numeric keypad while holding Alt in between each string.

Since ZAVT treats all console input the same, using cursor-up

as the full-screen key may preclude software such as 4DOS which

uses the key for its own purposes. In ZAVT without keyboard remapping

Ctrl-PgUp is used in its place as it is reasonably obscure but

still accessible. In ZAVT with keyboard mapping, there is no default.

When a key maps a double-null, that is taken as the hotkey. Thus

to define Ctrl-PgUp as the hotkey with ESCAPE.COM, type

c:>ESCAPE 0;132;0;0p

BUGS

The Set Mode and Reset Mode code 45 was chosen to match what

was used in NANSI 2.4 which in turn was chosen without even

looking to see if there were established codes for those functions.

Insert and delete character do not work in graphics modes

because BIOS does not include a scroll left/right function.

Cursor Position Report has a carriage return in it, which is what

ansi.sys does, but is nonstandard.

Avatar RLE doesn't handle control characters - it assumes whatever

you give it to repeat is printable and prints it, also assuming

the repeat will not result in wrapping past the right margin.

Wrap around left margin is nice, but nonstandard.

NOTE

While I polished off this code and added some of my own, the bulk

of it was written by Daniel Kegel, and Thomas Hanlin III to

whom credit is due. My distribution policy is the same as for the

formentioned authors' drivers, which is as follows as taken from

the documentation for NANSI24.

This program and all files associated with it are hereby placed

in the public domain, and thus may be freely copied and distributed.

If you really like this program, either

(a) improve it, and place your improvements in the public domain,

(b) write a neat program which uses Zavt's unique speed, and

place your program in the public domain,

(c) tip the author (Tip; contraction for "To Inspire Programmer";

traditionally, a contribution of $5 per copy), or

(d) resolve to become a better human being in the future.

Any problems or suggestions can be made to the 'author', Luns Tee

32 Thyra Ave.

Toronto, Ontario, M4C-5G5

Canada

FOR PROGRAMMERS

If the code seems to be a mess, that's because it is. Code from ZANSI

has been juggled around so as to minimize the number of JMPs that are

not JMP SHORT. Just a few hints:

Search for "90" in the .LST files. If any follow a JMP

instruction, make the JMP a JMP SHORT as these are NOPs to pad

the size difference between the two variants of this instruction.

When writing anything which writes to the console, use

an IOCTL call to get the driver's attributes, save them, and set

the console to raw mode. Do not write to the console by calling

Int 21h function 2 once for each character (it's not that uncommon)

but rather function 40h for each string. Buffering strings larger

than about 10 characters starts diminishing returns. Just remember

to return the console to its original state before exiting.

The file ZAVT.MAK has switches for Turbo Assembler and not

Microsoft MASM which NANSI was compiled under. The .MAK file itself

is for Microsoft's MAKE utility however.

FILES

ZAVT*.ASM	- console driver source files

ZAVT.MAK	- make file for ZAVT.SYS

ZAVT.SYS	- full-featured driver

ZAVTNK.SYS	- driver with keyboard remapping disabled

FONTS.ZIP	- assortment of ega/vga fonts

RAW.COM		- program to set console to raw

COOKED.COM	-   and cooked mode

ESCAPE.COM	- program to send escape sequences to console

ZAVT.DOC	- this file