In Windows/DOS Developer's Journal  December 1991, Robert Bybee wrote an

article entitled 'Writing For The PC Speaker'.  The program SPEAKER.EXE

is a butchered version of his program.

-The physics:

The PC can write individual levels to the speaker directly (a 1 or a 0).

It is also true that the PC can write these bits much faster than the speaker

can respond to them.  At these speeds, the speaker becomes an integrator.

-1-Bit DACs:

The latest craze in CD Players is 1-Bit dacs.  What is it?  Well...a

1-bit DAC really is not a DAC at all.  The sample is dithered (I will use

that term from here on out, even if it is wrong) with a bit pattern whose

integral (assume NRZL square wave) matches the level of the sample.  This

n-length bit pattern is sent one bit at a time into an integrator at a bit

rate of n (pattern length) times the sample rate.  From the integrator to the

pre-amp to the amp, etc...

For example a 3-bit sample would require 8 discrete levels, which would

require a 7-bit dither pattern:

dither           input

1111111     <--  111

1110111     <--  110

1101101     <--  101

1010101     <--  100

0101010     <--  011

0010010     <--  010

0001000     <--  001

0000000     <--  000

-Good quality PC Speaker sound:

Why not use the PC Speaker as an integrator for a 1-bit DAC?  Turns out to give

pretty good quality sound.  The example program does not attempt to find out

how fast your particular machine is.  You must adjust the bit pattern length

until it sounds right, or figure it out the machine speed yourself.  With my

486DX2-66 I was running so fast that the speaker was overwhelmed and the volume

was very low.  Adding a few NOPs to the end of the ophigh[] and oplow[] strings

should slow it down some.  This will give fewer bits per second which will give

better volume, but worse quality.

-Disclaimer:

This is not something I am going to develop further.