UART.COM is a memory resident utility that allows you to read and write the

com1 or com2 uart registers using a pop up that appears when you press

ctl-alt-lftshft.  It is a highly modified version of the printer setup util-

ity published by PC magazine and will not co-exist with the original utility

and also will not work with their dos-edit utility.  It will also conflict

with any programs that the original utility had a problem with.

Use is fairly much self-explanatory.

page up  -   selects com1 (beeps if no com1)

page dn  -   selects com2 (beeps if no com2)

lft arw  -   reads the register pointed to by the '>'

rgt arw  -   writes the byte at the btm of the pop up to the slctd reg

up/dn arw -  slcts register to read or write

0-9, A-F  -  hex entry of byte to write to a reg (appears at btm of pop up)

ESC       -  exit pop up

ctl-alt-lftshft - activate pop up

Registers were not all read and displayed when the pop up is opened due to

the fact that reading a register may affect the status of other registers

you wish to monitor.  Ex: Reading the rx holding register changes the data

ready bit of the line status register (this is not the only one that could

change the status of other regs or I would have wrote around it).

Included is the modified source also.  It was a quickie I put together

to use in debugging some async routines I was working on so don't look for

pretty code (its a mess but it works as long as it don't bump into some

of your other ram residents).  It works from within Codeview if you use that

and provides immediate access to the UART regs while debugging code.

It will also work from within your favorite com program and you can see

what interrupts they enable etc. but would not be a good idea to change

anything.  Following is quick summary of uart regs based on com1 address,

com2 is same except 2fx:

3f8 -  rx/tx hold reg (lsb of baud dvsr if bit 7 of LCR set)

(data in and out)

3f9 IER - interrupt enable reg (msb of baud dvsr if bit 7 of LCR set)

(who can interrupt?)

bit0 - enable rx data rdy intrpt

bit1 - enable tx hold reg empty intrpt

bit2 - enable receiver line status intrpts

bit3 - enable modem status intrpts

3fa IIR - interrupt identification register

(what caused interrupt?)

bit0 - set to ZR of interrupt pending

bits 2 & 1:

00 - ring indicator/line signal detect/modem status  interrupt

01 - tx hold reg empty caused intrpt

10 - byte ready in rx hold reg

11 - overrun/parity/framing error or line break received

3fb LCR - line control register

bits 1 & 0: number of data bits

bit2      : number of stop bits

bits 4 & 3: parity            (data,stop,parity bits same as for bios call)

bit5      : stick parity

bit6      : set break

bit7      : if high 3f8 & 3f9 are r/w regs for baud rate divisor (norm low)

3fc MCR - modem control register

bit0 - DTR (data terminal ready)

bit1 - RTS (request to send)

bit2 - output1 (not used by anything I've seen)

bit3 - output2 (set high to enable com port intrpts to intrpt controller)

3fd LSR - line status register

bit0 - data is ready to read from rx hold register

bit1 - overrun error occurred

bit2 - parity error occurred

bit3 - framing error occurred

bit4 - break was received

bit5 - tx holding reg is empty (rdy for another byte to tx)

bit6 - tx shft reg is empty

3fe MSR - modem status register

bit0 - DCTS delta clear to send (a chng in clr to send has been detected)

bit1 - DDSR delta data set rdy (chng in DSR detected)

bit2 - TERI trailing edge ring indicator

bit3 - DRLSD - delta received line signal detect

bit4 - CTS clear to send

bit5 - DSR data set ready

bit6 - RI ring indicator

bit7 - RLSD received line signal detect

Mike Dumdei               can be reached at:

6 Holly Lane               Split Up the Middle BBS   214 838-6713

Texarkana, TX 75503        Texarkana, TX