VGAUTILS.DOC FILE FOR VGA MONITOR SETUP/TEST UTILITIES:

VGRID.EXE, VGRID-M.EXE, VBOX.EXE, VGACOLOR.EXE

Contained in this "package" (the file VGAUTILS.ARC) are four

utilities that you can use to test and align your VGA display

adapter/monitor combination, along with this file, VGAUTILS.DOC.

VGRID.EXE, VGRID-M.EXE, and VBOX.EXE require a VGA or 100%

compatible display adapter and a VGA or VGA-compatible

multiscanning monitor.  They may work with certain "Super EGA"

cards with 640 x 480 16 color modes that are compatible with VGA

mode 18 (decimal) when used with a monitor capable of displaying

this resolution.  Beyond this, I cannot make any claims for the

compatibility of these programs with your equipment.

VGACOLOR.EXE requires a VGA or MCGA (or 100% compatible) display

adapter, and a VGA or compatible monitor.

These programs were written and compiled in Microsoft QuickBASIC

version 4.5.  They are to be considered as FREE software for

everyone's use and enjoyment.  The descriptions of the programs

are as follows:

VGRID.EXE

This program puts a white grid on your monitor screen in VGA mode

18 (decimal), -the 640 x 480 16 color graphics mode.  The outer

border of the grid is 640 pixels wide by 480 pixels high.  The

individual boxes of the grid are 40 pixels on a side.  This

screen pattern is useful for testing and aligning you VGA or

multiscanning monitor for the following:

FOCUS - The grid should, of course, be sharp all the way

across your monitor screen.  If it is not, the focus may

need to be adjusted.  Some poorly designed monitors may have

trouble maintaining focus in the corners when the center of

the screen is sharp.

VERTICAL AND HORIZONTAL SIZE - The accurate display of

graphics is important when using CAD or paint programs, -the

size and shape of displayed objects must be correct.  This

requires that the vertical and horizontal size controls of

your monitor be correctly set.  Since the borders of the

VGRID screen display measure the full 640 by 480 pixels

(width to height) of the VGA mode 18 screen, they should be

displayed at an aspect ratio of 4 units wide to 3 units high

(or 1.33 to 1, if you're so inclined).  If this is not the

case, adjust your monitor's vertical size control and/or the

horizontal size control for the correct screen aspect ratio.

You can set this by actually measuring the size of the grid

on the screen, or you can simply set your monitor so that

the individual boxes that make up the grid appear to be

square.  You can also set your monitor's screen height and

width visually using the program VBOX.EXE.

CENTERING - If your monitor's horizontal and vertical

position controls are set correctly, the grid display should

be centered on the screen.  Adjust these controls accord-

ingly if this is not the case.

HORIZONTAL AND VERTICAL LINEARITY - The boxes that make up

the grid display should (ideally) appear as perfect squares

of the same size across the entire screen display.  If they

are not all the same size, the horizontal and vertical

linearity adjustments of your monitor may be incorrectly

set.  Please note that it may be impossible to achieve

perfect linearity across the screen, -sometimes a compromise

setting may have to be determined to achieve the best

overall display quality.

CURVILINEAR DISTORTION - The horizontal and vertical lines

in the grid should appear perfectly straight.   They may,

however, be curved outward (barrel distortion) or inward

(pincushion distortion) in monitors that are out of adjust-

ment or just plain poorly designed.  Some monitors have

provisions to correct for these problems, others may not.

CONVERGENCE - In a color monitor, the three electron beams

for each of the primary colors (red, green, and blue) must

be aligned so that they meet ("converge") to form the

smallest possible point on the screen.  If convergence is

improperly set, there is a loss of sharpness in the form of

colored fringes on text and graphics.  The white grid

display of VGRID.EXE, when used in conjunction with the

magenta grid produced by VGRID-M.EXE, can be used to check

and set your monitor's convergence.

The VGRID display will remain on your screen until you press a

key.

VGRID-M.EXE

VGRID-M.EXE can be used along with VGRID.EXE to check and align

convergence in color monitors.  VGRID-M produces a display

identical to that of VGRID, except that the grid is magenta

instead of white.

The NEC Multisync II service manual recommends using magenta and

white "crosshatch" displays (such as those produced by VGRID-M

and VGRID) to set convergence.  The magenta grid display is used

first to set the alignment of blue and red.  There are tabs along

the neck of the monitor's CRT tube that move small permanent

magnets which control the convergence of these beams in both the

horizontal and vertical directions.  The idea is to get the

horizontal and vertical magenta lines to appear as sharp as

possible, without red or blue fringes.

Next, a white grid display (VGRID.EXE) is used to set the

convergence of magenta (red + blue) with green.  A different set

of tabs along the neck of the CRT are used this time, but the

principle is the same as that used in the red/blue alignment.

The white grid lines should appear as sharp as possible, with no

colored fringes.  When this condition has been obtained, all

three colors are properly converged.

The procedure for the convergence alignment of monitors other

than the NEC Multisync II should be similar.  It is recommended

that you obtain the service manual for your particular brand and

model of monitor, and adhere to its recommendations for setting

convergence and any other monitor adjustments.  Care must be

taken, as there may be control tabs near those used for con-

vergence that are used to set other things (such as color purity)

that you DON'T normally want to disturb.

CAUTION: Aligning convergence and other monitor settings usually

requires that you remove the cover from your monitor, thus

exposing yourself to the dangerous voltages that are inside.

Care must be taken to avoid electrical shock!  Nonconducting

plastic alignment tools (available at places like Radio Shack)

should be used.  Again, the best advice is to obtain the service

manual for your monitor, and follow its recommendations.

It may be impossible to obtain perfect convergence across the

full width of your monitor's screen.  Typically, convergence

error is greater in the corners.  As is the case with linearity,

a compromise setting may be necessary to obtain the best overall

alignment across the entire display width.

VBOX.EXE

This is a program that generates a screen display especially made

for setting the vertical and horizontal size controls of your

monitor.  The circles should appear to be perfectly round, and

the boxes should be perfectly square.  The border of the display

measures 640 pixels wide by 480 pixels high, so the screen aspect

ratio should be 4 to 3 (or 1.33 to 1).

VGACOLOR.EXE

This is a demo program that shows the capabilities of the

320 x 200 256 color mode (decimal mode 19) of the VGA or MCGA

display adapters.  The 256 colors used can be chosen from a total

"palette" of "256K" (actually 262,144) possible colors, which

vary in hue (shade), intensity (brightness), and saturation.

The colors of the palette are produced from all the possible

combinations of the three primary colors (red, green, and blue),

each of which can be varied through 64 steps of intensity (64 x

64 x 64 = 262,144).  On monochrome monitors, these colors are

displayed as 64 shades of gray.

The first screen shows the "default" color settings of the 256

screen color attributes.  The numbers and letters along the top

and left side of the color bar display are hexadecimal numbers.

You can determine the hex attribute number of a displayed color

bar by using the numeral displayed to the left of its horizontal

display row as the first digit, and the numeral directly above

its vertical display column as the second digit.

The top row (attributes 00H to 0FH) of the first screen is the same

as the "default" color set used by the EGA.  The second row (10H

to 1FH) shows the range of 64 gray levels possible in 16 evenly

spaced steps.  Attributes 20H through F7H consist of 24 color

groups, each consisting of nine shades of a color.  Three

intensity levels and nine saturation levels are provided for each

shade.  Attributes F8H through FFH are set to black for reasons

known only to the designers of the VGA standard.

The second screen uses attributes 20H through FFH to show how the

colors used can be varied in hue (shade) and intensity (bright-

ness).  Hue is varied along the vertical axis of the display.

The primary colors (red, green, blue), secondary colors (yellow,

magenta, cyan), and some intermediate shades are shown.   The

"spacing" of the hues shown is not "even" in the sense of the

numerical values of each color, but were chosen to visually

present discrete "steps" of hue variation.  Exactly how

this looks to you will depend on the contrast and brightness

settings of your monitor, as well as your own eyes.  The 64 step

range of intensity/brightness for each displayed hue is varied

from left to right in 16 evenly spaced steps ranging from "0"

(black) to full intensity.

The third screen uses attributes 20H through FFH to show how

colors can be varied in saturation.  As in the previous screen,

hue is varied in the vertical axis.  The 64 step range of color

saturation is shown from left to right in 16 evenly spaced steps

ranging from "no saturation" (pure white) to full color

saturation.

VGACOLOR, besides being a demonstration of the capabilities of

VGA/MCGA mode 19 (decimal) can be useful for setting the

brightness and contrast settings of your monitor.  It is also

an aid in convincing your spouse that the hundreds of dollars

that you spent on a VGA adapter and monitor were worth it.

NOTES ON MONITOR VERTICAL/HORIZONTAL SIZE SETTINGS

Depending on the brand and model of your display adapter and

monitor, the display vertical and horizontal size settings that

you determine using VGRID.EXE or VBOX.EXE may only be valid for

the VGA 640 x 480 mode.  Other screen resolutions may require

that the monitor be readjusted for the proper display aspect

ratio.

The reason for this is that different monitors can behave in

dissimilar ways as the video horizontal and vertical scan rates

are varied for different on-screen resolutions.  Normally, if

nothing is done by the monitor to compensate for these changes,

the size of the displayed image will vary considerably with scan

frequency.

Normally, this is not a problem with the standard VGA display

modes and VGA-only monitors.  The VGA standard specifies a

constant horizontal scan frequency of 31.5 KHz regardless of the

video mode, with a vertical scan rate of either 60 or 70 Hz.

Most VGA display adapters adhere to this, and most VGA monitors

expect this range of scan rates, and all is well.

However, problems can arise with some of the non-standard,

higher resolution "Super VGA" modes (800 x 600, 1024 x 768, and

the like) found in some display adapters when those modes are

used with the "multiscanning" or "multisync" monitors needed to

display the higher resolutions.  In the Super VGA modes, the

horizontal scan rate may be as high as 35 KHz, while the vertical

scan frequency may be as low as a flickery 50 Hz (this is done to

keep the overall video bandwidth down to something most monitors

can handle).  Different monitors deal with these changes in

horizontal and vertical scan frequencies in various ways.  Most

multiscanning displays have some sort of "autosizing" circuitry

built into them to maintain a constant image size despite changes

in scanning rates.  In some of these monitors, the autosizing

feature works very well, and once you have set the width and

height for one display standard, it is also correctly set for

other modes as well.  Other monitors have autosizing circuits

that do not work as well, or they may lack this feature entirely.

With these displays, you may have to reset the screen vertical

and horizontal size controls when switching between different

resolutions and display modes.

It is also possible that the centering of the image on your

monitor screen may vary with the resolution being displayed.

This is particularly true for the horizontal image position.

Some multiscanning monitors (such as the NEC Multisync II) have a

separate horizontal image centering control for each of several

horizontal scan frequency ranges.

QUESTIONS AND COMMENTS

You can leave questions or comments regarding these programs for

me on CompuServe (73710,2014).

-Arthur Stoppe

July, 1989