FX - TW-523 Special Effects Interpreter - Version 1.00

Copyright 1992-93 by Bruce Christensen. All Rights Reserved.

==============================================================================

Introduction:

==============================================================================

FX.EXE is a DOS-based X10 command interpreter for the TW-523.  Valid

X10 commands are ON, OFF, DIM and BRIGHT. These commands can be input

from the DOS command line, or they may be read from a "script" file.

If you are familiar with XA (X10 Command Interpreter for the CP-290),

then you should grasp the concepts of FX very easily. If you are not

familiar with XA, and you own CP-290, then you should download XA to

expand the capabilities of your CP-290.

A special feature of FX is its ability to send SIMULTANEOUS Bright

and Dim commands to multiple X10 modules. This allows you to create

some very interesting lighting effects.  My primary use for this

program is for Halloween and Christmas lighting.

FX communicates with the TW-523 via your computers parallel port

(LPT1...LPT3).  See the section "LPT Wiring" for the I/O lines FX

uses to communicate with the TW-523.

This document is filled with many examples. Most examples use the

command line to issue commands. Complete commands from the DOS

command line must be enclosed in "quotation marks":

FX "A1 ON"

However, when commands are read from a script file, they are not

enclosed in quotation marks:

A1 ON

==============================================================================

Christmas Lighting:

==============================================================================

Our indoor Christmas tree, as well as several outdoor trees are

decorated with multiple strings of solid color mini-lights: RED,

BLUE, GREEN, WHITE.  Each color is controlled by a Lamp/Dimmer module

with a unique house code:

Lamp/Dimmer controlling RED   : L1

GREEN : M1

BLUE  : N1

WHITE : O1

The intensity of each string may be continuously varied so that the

trees are always changing colors. The really slick effect is when you

DIM one color and BRIGHTEN another color at the same time. Your tree

will change from Red to Blue (or whatever you program) in about a

second.

I've included a command file (XMAS_FX.CMD) to demonstrate how I

program these effects. The rest of this document explains the command

line arguments and language syntax of FX.

==============================================================================

FX Syntax and Arguments:

==============================================================================

FX Syntax:

--------------------------------------

FX is invoked by typing FX along with any of the following arguments.

These arguments are optional. The default value will be used for non-

specified arguments.

FX [f=script_file] [L=1...3] [o=xxx] [-e] [+h] ["command"...]

Arguments:

--------------------------------------

[f=script_file]

This is the name of a file containing commands that are read

by FX and sent to the TW-523. To have FX read the example

XMAS_FX.CMD script file:

FX f=XMAS_FX.CMD

If no "commands" are present on the command line, and no script file

is specified, then FX will attempt to read the file FX.CMD (default).

[L=1..3]

Specifies the LPT port that the TW-523 is connected to. There are

three possible LPT ports supported in standard PC configurations.

LPT     I/O addr

===     ========

1       0x378

2       0x278

3       0x3BC

On some machines (including combination monochrome display adapters

and printer cards), LPT1 uses I/O port 0x3BC.

The default port is 1 if this option is not specified.

[o=xxx]

Specifies a special I/O port address. Use this parameter if your

LPT I/O address is something other than one of the values located

in the above table. The default address is 0x378.

-e

This option will prevent FX from sending any commands to the TW-523.

-h

+h

Displays a "usage" screen listing available commands, syntax, and

the FX version level.

["commands..."]

FX will execute commands from the DOS prompt, or from within

batch files. Commands must be enclosed in "quotation marks"

when the are executed in this manner.

FX "A1 OFF" "B1 OFF" "C1 OFF"

==============================================================================

FX Tokens

==============================================================================

This section describes each of the tokens that FX understands.  Most

commands are composed of multiple tokens. In certain cases, the order

of the tokens is not important. For example, the command:

FX "A1 ON"     may also be written as:

FX "ON A1"

In other instances, the tokens must be arranged in groups. Their order is

important. For example, the command to dim a module is "DIM #levels".

You would therefore program the command as:

FX "A1 DIM 20"

FX "DIM A1 20" <--- This is illegal.

Therefore, token groups dependent on order will be enclosed in curly

braces { }.

{DEFINE X Y}

--------------------------------------

This token grouping allows you to substitute a text string (X) for another

text string (Y). For example, to assign a module (addressed as L1) a name

such as RED, you would define it as follows:

DEFINE RED L1

(X) (Y)

Now, instead of issuing a command such as:

FX "L1 ON"    you can use a more natural command:

FX "RED ON"

A SPACE character separates the X from the Y string. The Y string can have

multiple spaces. If you want to use multi-word strings for X, use the

UNDERSCORE character:

DEFINE LOW DIM 0

X  <-Y->

DEFINE XMAS_TREE E2

<---X--->  Y

The DEFINES used in the XMAS.CMD file are listed below. These DEFINES will

be used in examples throughout this document.

DEFINE RED    L1

DEFINE GREEN  M1

DEFINE BLUE   N1

DEFINE WHITE  O1

DEFINE PC     C4

DEFINE PORCH  A9

ON

--------------------------------------

Turns ON the specified module.

FX "A1 ON"

OFF

--------------------------------------

Turns OFF the specified module.

FX "B1 OFF"

{DIM #levels}

--------------------------------------

DIMs the specified module by # levels. There are approximately 18

levels of brightness supported by X10 wall switch/lamp dimmer modules.

The '#levels' tells FX how many times to send the DIM command to the

module. The minimum value must be 1, the maximum value can be any positive

number greater than 1.

To dim module C2 by 10 levels from its current level:

FX "C2 DIM 10"

Over time, a module may not be at its expected brightness level when

multiple DIM (or BRIGHT) commands have been sent to it. This may be

caused by lost or garbled transmissions from the TW-523. To offset

this possibility, when dimming a module to its completely dimmed

state, set the number of levels to a higher than normal number to

ensure the module is completely dimmed:

FX "D4 DIM 25"

{BRIGHT #levels}

--------------------------------------

BRIGHTens the specified module by # levels. There are approximately 18

levels of brightness supported by X10 wall switch/lamp dimmer modules.

The '#levels' tells FX how many times to send the BRIGHT command to the

module. The minimum value must be 1, the maximum value can be any positive

number greater than 1.

To brighten module A5 by 6 levels from its current level:

FX "A5 BRIGHT 6"

Over time, a module may not be at its expected brightness level when

multiple BRIGHT (or DIM) commands have been sent to it. This may be

caused by lost or garbled transmissions from the TW-523. To offset

this possibility, when brightening a module to its completely bright

state, set the number of levels to a higher than normal number to

ensure the module is completely brightened:

FX "A1 BRIGHT 25"

Additional BRIGHT / DIM Examples

--------------------------------------

Assuming the RED lights (module L1) are fully on (100% brightness), to dim

them by 25%:

RED DIM 5

If BLUE is completely DIM (0% brightness), to brighten them to 50%;

BLUE BRIGHT 10

If GREEN is fully bright and you want them completely dimmed:

GREEN DIM 20

Simultaneous Bright/Dim Commands

--------------------------------------

Simultaneous commands may only occur with the BRIGHT and DIM tokens.

In addition, the modules controlled simultaneously are required to

have unique housecodes. For example, you could control A1, B1, C1,

D1...P1 simultaneously in one command. You CAN NOT control A1, A2, A3

simultaneously in on command.

You must use the "+" (plus) symbol between multiple commands to tell

the FX interpreter that these commands are to be treated as

simultaneous. Once a module is commanded to brighten or dim, it

continues that command no matter what subsequent modules are issued

for the rest of the command.

BLUE BRIGHT 1 + GREEN DIM 20

In the above example, BLUE is issued a command to brighten by 1

level.  Immediately following the bright command, GREEN is dimmed by

20 levels, however BLUE continues to brighten for the duration of the

entire command.  Therefore BLUE receives a total of 21 BRIGHT

commands, GREEN receives 20 DIM commands.

You can delay the effects of simultaneous transmissions by altering

the "levels" of each command. Assuming all lights are fully on, the

following command will start fading WHITE 5 levels, then begin fading

RED 5 levels (WHITE continues fading), then GREEN, then BLUE. Note

that BLUE uses 20 levels to reach its desired state - the other

modules recieve more commands than actually needed to get to the

desired state.

WHITE DIM 5 + RED DIM 5 + GREEN DIM 5 + BLUE DIM 20

A module is NOT allowed to receive different commands on the same line:

RED DIM 5 + BLUE BRIGHT 5 + RED BRIGHT 10

See the file XMAS_FX.CMD for more examples of simultaneous programming.

{DELAY seconds}

--------------------------------------

DELAY allows you to wait "seconds" before continuing on with the rest

of the program. To wait 5 seconds between commands:

RED BRIGHT 1 + WHITE BRIGHT 1 + GREEN DIM 1 + BLUE DIM 20

DELAY 5

WHITE DIM 20

Support for program looping

--------------------------------------

You may repeat a series of commands as often as you like by using the

"GOTO label" and ":label" tokens. The ":label" token identifies a

location in the file that FX may reference with a GOTO command later

on. The label must have been parsed prior to the GOTO token.

The GOTO command will jump to a previously defined label. You may

specify the number of times the jump is to be performed. To repeat a

sequence of instructions 5 times:

:HERE

RED DIM 1 + BLUE BRIGHT 20

BLUE DIM 1 + RED BRIGHT 20

GOTO HERE 5

Following the 5th pass of the GOTO statement, FX continues with the

remaining commands in the command file.

If you do not specify a repeat value, FX will continue to execute the

GOTO statement inedefinately:

:REPEAT

RED DIM 1 + BLUE BRIGHT 20

BLUE DIM 1 + RED BRIGHT 20

GOTO REPEAT

The :EXIT label has special meaning to the FX interpreter. FX will

always search for :EXIT whenever the <ESC> key has been detected.

This allows you to create a controlled shutdown command sequence

ensuring certain modules are in a desired state.

:EXIT

RED OFF

GREEN OFF

BLUE OFF

WHITE OFF

PORCH BRIGHT 20

PC OFF

If :EXIT is not present, FX will simply stop what it is doing and

return to DOS.

{TIME hh:mm [AM PM]}

Controlled Shutdown Time

--------------------------------------

The PC I use to control the lighting sequences is activated at Dusk by

the CP-290. Once this PC is booted it begins transmitting X10

commands via the TW-523 module. This PC saturates the powerline with

continuous X10 commands leaving virtually no bandwith for the CP-290

or other transmitters to transmit clean X10 commands to shut the PC

down later on.

Using TIME hh:mm at the beginning of the script file will ensure that

FX stops executing at a specified time:

TIME 11:30 PM

After every X10 command is issued, the FX interpreter will compare the

current time with the preset time. When the times match, the shutdown

portion of the script file is executed if the :EXIT token is located.

If :EXIT is not included, then FX simply stops and returns to DOS.

==============================================================================

LPT Wiring

==============================================================================

The TW-523 was developed to allow vendors to interface their

equipment with X10's patented communication protocol. It was not

intended to be sold to consumers, therefore there is no "standard"

configuration for attaching the TW-523 to your computer. There are

various kits that offer serial and parallel port connectors, but

these kits may differ from vendor to vendor.

The software used in FX is only capable of handling LPT communications

with a 25 pin connector using the following pinout diagram. This pinout

is compatible with the TW-523 kit sold by HCC (although that is subject

to change).

PC's LPT port                                         TW-523 RJ

(25 Pin)                                         connector

=================                                    =============

Pin   2 -+

|

4 -+

|

6 -+

|     (Pins 2,4,6,8 are tied together)

8 -+------->>-------(Transmit)------>>---------- 4

10 -+

|     (Pins 10 and 14 are tied together)

14 -+-------<<-------(Receive)-------<<---------- 3

(These lines not currently used)

25 -------------------(Ground)------------------- 2

17 ---------<<---------(60hz)--------<<---------- 1

Radio Shack sells a 25-pin Male D-shell kit with a 6-pin RJ connector

that allows you to create the LPT pinouts as described above.

A final note on parallel ports - some manufacturers of I/O cards with

LPT ports do not support all the I/O data lines as specified by

IBM. This is most noticable on some very early I/O cards. If you

experience problems such as "No AC sync signal detected", try a

different LPT port.

==============================================================================

Other X10 Programs by Bruce Christensen

==============================================================================

XA - X10 Command Interpreter

--------------------------------------

XA translates English-like sentences into commands or events that are

understood by the CP-290. XA receives its input directly from the DOS

prompt (XA "A1 ON") or it can read an entire script from a file (XA

F=XA.CMD).

XA can maintain your CP-290's event schedule automatically. XA comes

with a utility that allows new events to be downloaded on a weekly

basis without any interaction from a user.  XA calculates SUNRISE and

SUNSET times based on your latitude and longitude, and automatically

compensates for any Daylight Savings changes. XA can be programmed to

handle specific dates that may be weeks, months, or years in the

future.  Events may also be excluded on certain dates.

XA also supports a power-fail recovery mode which restores all

programmed modules to their expected state. XA supports the CP-290

"undocumented" commands ALL_LIGHTS_ON and ALL_UNITS_OFF. XA allows

you to append new events to your existing CP-290 schedule easily from

the DOS command line. XA can synchronize either your PC or CP-290

internal clocks to an accuracy of 1 second. XA can monitor (display)

events as the CP-290 executes them, it can also log these events to a

file.

XA is the most powerful shareware program available for the CP-290.

You will never need to use the original X10-supplied software again.

When you register your copy, you also get enhanced programming

capabilities such as IF/ELSE constructs, Logical/Arithmetic/Boolean

operators, user variables, and the ability to interface with I/O such

as the joystick port.

The current version of XA is 2.00. It has been uploaded to numerous

BBS's, including: CompuServe, America OnLine, PC-Ohio, HAL, Baran-

Harper, Circuit Cellar.  Look for the file: X10XA200.ZIP.

XT - X10 TSR

--------------------------------------

XT is a Terminate-Stay-Resident program that sits silently in the

background waiting for you to activate it with the proper keystroke.

This means you can instantly call it from within any text-based

application. When activated, it displays a list of all modules that

you can control with the CP-290. Each line on the list contains a

description of the module, its address, and its current status.  You

select the module you want to control, and then choose the action

(ON, OFF,DIM).

Since XT is resident, it continuously monitors the serial port for any

communication transmitted by the CP-290 (such as when an event is

issued or when one of the buttons on the unit is pressed). The status

of the each module will always be reflected in the pop-up menu.

The current version of XT is 1.00. It has been uploaded to numerous

BBS's, including: CompuServe, America OnLine, PC-Ohio, HAL, Baran-

Harper, Circuit Cellar.  Look for the file: X10_TSR.ZIP.

A newer version is planned for release during January 1994. This

version of XT, (1.02) will come with a utility called XTS.EXE. XTS

has direct access to XT's database of module states. Every command

issued by the CP-290 is captured and stored by XT. XTS can return the

module states (via DOS 'errorlevel' statements) allowing you to write

batch files that react to a modules X10 status.

Look for file: X10XT102.ZIP.

==============================================================================

Registration

==============================================================================

FX is a copyrighted program. Please support shareware and register this

software. Requested fee is $10.00 - US funds.

Please describe how you received the program and what your primary

use is. Also include any comments or suggestions for new releases.

When distributing this program to other users, please leave all the

original files intact.

Author Information:

--------------------------------------

Bruce Christensen

6594 Hudson Ave.

Mentor, OH.  44060-4545  USA

CompuServe     : 73201,1531

America On-Line: AuggieBen

Prodigy        : MHNC93A