======  Device Driver Update Note       Ver. 2.20  ===========================

Device driver supports the following functions.

1.Supports maximum 8 drives.(HITACHIA.SYS only)

If you use more than 4 drives by one device driver, you need to

use an interface card and drives that support 8-drive-daisy-chain.

(e.g. CDR-IFI8-A, CDR-3600, etc.)

2.Supports power-save function.

Power-save function is designed to make your drive's life longer.

When power-save mode is set and the drive doesn't receive any

command from the host for about 9 minutes, it will automatically

spin down and wait for command from the host.   If the drive receives

some command in this stand-by mode, it will soon spin up (in a few

seconds) and execute the command.

The device drivers (HITACHIA.SYS and HITACHIB.SYS) will set power-save

in default.

If you do not want to use this function, put '/C' on the command line

switch of the device driver in this "CONFIG.SYS" file.

Example:

DEVICE=\DEV\HITACHIA.SYS /D:MSCD001 /N:1 /C

3.If you change the clock speed on any CPU, cache controller setting on an

80486 CPU, or CD-ROM dive setting( drive No., number of drives etc.),

please reboot the system.   When the device driver is loaded at boot

time, it will recalculate its algorithm for transferring data from

the CD-ROM drive at the most effective rate for that system

configuration.

#