Device Driver INFORMATION

-------------------------

This README file contains some additional information to supplement that

contained in the User's Manual.

-----------------------  ADDITIONAL INFORMATION  -----------------------

1. SCSI interface borad

The DRD600.SYS is the device driver for FUTURE DOMAIN SCSI interface

board (8bit, 16bit and microchannel) or it's compatible (PIONEER DD-C5002).

The DRD60PS.SYS is the device driver for IBM PS/2 SCSI Adapter /A.

Please use the appropriate driver for your interface board.

2. DRD60ASP.SYS and ASPIFD.SYS (in ASPI directory)

The DRD60ASP.SYS device driver is an ASPI version of the DRD series device

drivers for the Pioneer DRM-600.

3. DISC.EXE utility

The DRD600 series device driver can handle a maximum of 12 Discs.

Some of the currently available applications can only work on systems

that have a desiginated drive letter and subunit number. The DISC.EXE

utility program allows the desigination of the CD-ROM subunit No. to

be in any desired order, but only in conjunction with theDRD600

series Device Driver.

The use of this program is explaind below;

To see the current order of the Discs.

>DISC cd-rom_device_name

>DISC cd-rom_drive_letter:

The drive letter is required ':'.

To change the current order of the Discs.

>DISC cd-rom_device_name [-R] no1 no2 .....

The position of the number to be changed is switched with the

position of the number its replacing. Using the -R option reseting

of the order from 0 upwards is possible.

To exchange two drive letters.

>DISC cd-rom_drive_letter: cd-rom_drive_letter:

4. DRDUTL.EXE utility

The DRD600 series device driver and the MSCDEX.EXE allows the MS-DOS

to add multiple CD-ROM drives. Currently, some applications will waste

time do useless actions or can't handle the device driver error messages,

if multiple CD-ROM drives are connected. This utility changes the driver's

behavior (mode) to permit the optimum performance of this type of

application. Use this utility only when your application program requests

this kind of change of mode.

The use of this program is explaind below;

To see the current mode.

>DRDUTL CD-ROM_device_name

>DRDUTL CD-ROM_drive_letter:

The drive letter is required ':'.

To change the current mode.

>DRDUTL CD-ROM_device_name switch [switch] ..

>DRDUTL CD-ROM_drive_letter: switch [switch] ..

switch

DSKCHK:ON(OFF) Enables or disables the disc check operation.

SEEK:ON(OFF)   Enables or disables the seek operation.

VDRTRY:ON(OFF) Enables or disables "Invalid Medium Change"

error message when reading the Volume Descriptor.

Follow the setting of these switches as the application request.

5.SETUP.EXE

To install DRD600.SYS, DRD60PS.SYS and MSCDEX.EXE,the setup program is

the most convenient method. Please be aware, that if an EMS buffer has

been allocated the device driver switches will have to be changed.

Refer to the user`s manual for further details.

-----------------------------  ATTENTION  -----------------------------

1. Please make sure that the DRM-600 or DRM-604X drive's SCSI ID number,

which is set by dip switches or selector at the rear of the changer, and

the other SCSI device's (eg. SCSI hard disk or MO disk) ID numbers are

not the same. This is to ensure that the system will operate correctly

when power is turned on. For more detailed information on the SCSI

device's ID number,please consult your SCSI device's user's manuals.

2. WARNING, in the interest of safety and of possible harmful side

effects to the devices that are to be interconnected. Please ensure that

power to the devices is turned off before attempting to connect DRM-600

or DRM-604X to a computer.

3. The IBM SCSI Adapter /A interface board checks the SCSI devices when

the computer is booted up. If the SCSI device doesn't answer correctly,

the device isn't available once the computer has booted up. Please turn

the power of the drive on before or at the same time as that of the

computer.

----------------------------  VERSION UP  -----------------------------

1. DRD600.SYS,DRD60PS.SYS  Ver.1.10

If the "/s"(set SCSI ID) is omitted, the SCSI bus is automatically

searched for an available SCSI ID for up to 2 connected CD-ROM drives.

If the switch "/s" is used, the automatic SCSI ID search feature is

over-ridden and the provided SCSI ID is used.

2. The DRD600.SYS device version has been upgraded to 1.10.

The 16 bits FUTURE DOMAIN SCSI interface board has been supported.

3. The MICROSOFT CD-ROM EXTENSIONS (MSCDEX.EXE) has been upgraded to 2.21.

This version of MSCDEX works with EMS Ver.4. Thus Windows Ver.2.11 or

Ver.3 real mode will work with this version of MSCDEX.