;==========================================================

DC-820 / DC-320 Programming interface

;==========================================================

;==========================================================

I. I/O ports

;==========================================================

Possible I/O	  : 330, 334, 230, 234, 130, 134

Slot specific I/O : XC80--XC9F

: X = slot #, 1 --> F

XC80--XC83 == EISA BOARD ID

IO/PORT -->	XC80	XC81	XC82	XC83

--------------------------------------------

DC-820		52	4D	08	20

DC-320		52	4D	03	20

XC8D <-- 2 ==> Out 2 to port XC8D to start single threaded command

XC8F != 0 ==> Command complete status

XC90--XC9F == Command block

;==========================================================

II. Single thread command

;==========================================================

(A) Protocol

Out FF to XC8F

Fill XC90--XC9F with command block

Out 2 to XC8D to start command

wait for XC8F != 0 ==> command complete

XC90 = execution status

When XC90--XC9F is given the command block

XC90 represent the command OPCODE

XC91--XC9F = command parameter

;==========================================================

command 0 : reset SCSI	( BIOS function )

input : 0

output : 0

;==========================================================

command 1 : read status  ( BIOS function )

input : 1

output : 0

;==========================================================

command 2 : read sectors  ( BIOS function )

input : 2, SCSI-ID(1), block count (1), block address(4), buffer address(4)

output : status(1),error code(1)

;==========================================================

command 3 : write sectors  ( BIOS function )

input : 3, SCSI-ID(1), block count (1), block address(4), buffer address(4)

output : status(1),error code(1)

;==========================================================

command 4 : verify sectors  ( BIOS function )

input : 4, SCSI-ID(1), block count (1), block address(4)

output : status(1),error code(1)

;==========================================================

command 5 : invalid

;==========================================================

command 6 : invalid

;==========================================================

command 7 : invalid

;==========================================================

command 8 : read drive capacity    ( BIOS function )

input : 8, SCSI-ID(1)

output : status(1), drive capacity(4)

;==========================================================

command 9 : ignored

;==========================================================

command A : invalid

;==========================================================

command B : invalid

;==========================================================

command C : ignored

;==========================================================

command D : reset SCSI	( BIOS function )

input : D

output : 0

;==========================================================

command E : invalid

;==========================================================

command F : invalid

;==========================================================

command 10 : ignored

;==========================================================

command 11 : ignored

;==========================================================

command 12 : invalid

;==========================================================

command 13 : invalid

;==========================================================

command 14 : ignored

;==========================================================

command 15 : read drive capacity   ( BIOS function )

input : 8, SCSI-ID(1)

output : status(1), drive capacity(4)

;==========================================================

command 16 : invalid

;==========================================================

command 17 : invalid

;==========================================================

command 18 : mirror option    ( TOOLBME : Used for DISK IMAGE COPY )

input : 18, 1/0 (1)	1:enable the previously disabled mirror pair

0:disable ALL mirror operation

output : 0

;==========================================================

command 19 : read errors	NO FUNCTION

;==========================================================

command 1A : write cache data to Disk  ( used by TOOLBME )

input : 1A, SCSI-ID(1)		ID=0--7:write cache with drive ID

ID=8:write all cache data to drive

output : 0

;==========================================================

command 1B : invalidate cache	( Used by TOOLBME )

input : 1B, SCSI-ID(1)		ID=0--7:invalidate that ID, ID=8:invalidate all

output : 0

;==========================================================

command 1C : cache control	( Used by TOOLBME )

input : 1C, control(1)

output : 0

control byte = 0..7 ==> disable the specified device cache option if it is cacheable

80..87 ==> Re-enable the specified device

8    ==> Disable all the device

88   ==> Enable all the device

;==========================================================

command 1D : check cache

input : 1D, SCSI_ID(1)		ID=0--7:SCSI ID to be checked

output : 0/1			0:cache is clean, 1:cache is dirty

;==========================================================

command 1E : scatter gathered read sectors	( BIOS function )

input : 1E,SCSI-ID(1),block count(1),block address(4),address list address(4),

address list entries(2)

output : status(1),error code(1)

;==========================================================

command 1F : scatter gathered write sectors	( BIOS function )

input : 1F,SCSI-ID(1),block count(1),block address(4),address list address(4),

address list entries(2)

output : status(1),error code(1)

;==========================================================

command 20 : controller setup, USED BY EISA SYSTEM BIOS only to INITIALIZE

THE CONTROLLER ( USER APP. DO NOT USE THIS FUNCTION!!! )

input : 20,s1,s2,s3,s4,s5,s6,s7,s8,s9,s10,s11,s12,s13,s14,s15

output : 0

s1 bit 543	IRQ

000	15

001	15

010	14

011	12

100	11

101	10

110	9

111	disabled

s1 bit 210	BIOS address

000	DC00

001	DC00

010	D800

011	D400

100	D000

101	CC00

110	C800

111	disabled

s2 bit 210	I/O PORT

000	330

001	334

010	230

011	234

100	130

101	134

110	disabled

111	---

s2 bit 543	DMA CHANNEL

000	0

001	DISABLED

010	---

011	---

100	---

101	5

110	6

111	7

S3 = cache line size, possible value = 16,8,4

s4 = define adapter SCSI-ID, possible value = 0,1,2,3,4,5,6,7

s5 bit	32	DRAM test

00	suppressed

01	simplified test

10	full test

11	---

s5 bit	10	DOS 5 option

00	---

01	disabled

10	enabled, use interrupt vector ( NO FUNCTION )

11	enabled, steal 1K from DOS

s6 power on delay for testing drive ready

possible value

20,30,40,50,60,80,100,120	seconds

s7 reserved

s8	hash depth	( NO FUNCTION )

s9	reserved

s10	reserved

s11	reserved

s12	reserved

s13	reserved

s14	reserved

s15	reserved

;==========================================================

command 21 : read EEPROM	( BIOS/DRIVER/TOOLBME Initialization )

input : 21, buffer address(4)

output : 0 ( Refer to paragraph : EEPROM FORMAT )

;==========================================================

command 22 : write DRAM 	( BIOS initialization )

input : 22, block count(2), block address(2), buffer address(4)

output : 0

;==========================================================

command 23 : read DRAM		( BIOS initialization )

input : 22, block count(2), block address(2), buffer address(4)

output : 0

;==========================================================

command 24 : write EEPROM	( Configuration or ACF20SU to set options )

input : 24, buffer address(4)

output : 0

;==========================================================

command 25 : invalid

;==========================================================

command 26 : invalid

;==========================================================

command 27 : invalid

;==========================================================

command 28 : invalid

;==========================================================

command 29 : invalid

;==========================================================

command 2A : invalid

;==========================================================

command 2B : invalid

;==========================================================

command 2C : invalid

;==========================================================

command 2D : set DRAM size	( BIOS Initialization )

input : 2D, dram size(2)	; in Kbytes

output : 0

;==========================================================

command 2E : get drive information	( BIOS/Driver Initialization )

input : 2E, SCSI-ID(1), buffer address(4)

output : 0

buffer : 36 bytes inquery data

4 bytes capacity

4 bytes sector size

;==========================================================

command 2F : invalid

;==========================================================

command 30 : diag 0		( Testing only )

input : 30, control(1)		set BIOS address and IRQ selection

output : 0

;==========================================================

command 31 : diag 1		( Testing only )

input : 31, control(1)		generate IRQ to host

output : 0

;==========================================================

command 32 : diag 2		( Testing only )

input : 32, control(1)		set I/O address

output : 0

;==========================================================

command 33 : diag 3		( Testing only )

input : 33, data(15)		invert 15 bytes of data

output : 0, data(15)

;==========================================================

command 34 : diag 4		( BIOS initialization )

input : 34, buffer address(4)	; Read firmware version string

output : 0

;==========================================================

command 35 : diag 5		; read 64K bytes of SRAM	( Testing only )

input : 35, buffer address(4)	; assume buffer address is 4 byte boundary aligned

output : 0

;==========================================================

command 36 : diag 6

;==========================================================

command 37 : diag 7

;==========================================================

command 38 : diag 8

;==========================================================

command 39 : diag 9

;==========================================================

command 3A : diag a

;==========================================================

command 3B : Controller reset	; Controller is reset, system must be RESET

input : 3B , 12, 34		; Used after writing controller EEPROM

output : None

;==========================================================

; EEPROM FORMAT

;==========================================================

command 21 READ EEPROM ==>

EEPROM data format

Byte 0	 --> 127 = EEPROM data

byte 128 --> 143 = SETUP data and tha same as that of command 20H

;==========================================================

Each ID has 8 bytes options

==> Only first 64 bytes of EEPROM is used in DC-820/DC-320 controller

byte0	bit 10		00	Non-cacheable device

01	Cacheable device

32		00	Active sync nego

01	Passive sync nego

1x	Async

4		0	Parity check enabled

1	Parity check disabled

5		0	Allow disconnect/reconnect

1	Do not allow disconnection

76		00	BIOS device if HD

01	BIOS device if HD or removable media

1x	Not BIOS device

byte1	bit 3210	0	1G	64x32		; Traslation method

1	1.5G	64x48		; For BIOS Controlled

2	2G	128x32		; Disk device

3	3G	128x48

4	4.5G	192x48

bit 7654	1000	Mirror master or non-mirror device

0xxx	Mirror slave

xxx=0--7 : Mirror master's SCSI-ID

byte2	bit 210 	0	10  MB/SEC	; fast SCSI

1	8   MB/SEC	; fast SCSI

2	6.6 MB/SEC	; fast SCSI

3	5.7 MB/SEC	; fast SCSI

4	5   MB/SEC	; normal SCSI

5	4   MB/SEC	; normal SCSI

6	3   MB/SEC	; normal SCSI

7	2   MB/SEC	; normal SCSI

;==========================================================

; DOS 5 support parameter table ( 16bytes )

;==========================================================

4 bytes : previous INT13 or previous parameter table

1 byte	: previous 475H ( 80h--86h )

1 byte	: my total devices ( 81h--88h )

2 bytes : Slot specific I/O address

7 bytes : low nibble SCSI-ID, high nibble translation method

1 byte	: bit 7 = 1 link to INT13

0 link to parameter

low nibble = I/O address bit 0123 (OR) bit 89AB

0000	---

0001	130

0010	230

0011	330

0100	---

0101	134

0110	234

0111	334

;==========================================================

;==========================================================

; BIOS controlled device detect

;==========================================================

;==========================================================

All BIOS functions are Adaptec compatible except when DOS 5 is enabled

==> When DOS 5 support disabled ( less or equal to 2 drives support )

AH=6	---

CY ==> No AHA compatible BIOS controlled device

NC, BX=80	first device = ID 0, second device = ID 1

NC, BX=81	first device = ID 0

==> When DOS 5 support enabled ( greater than 2 drives support )

AH=6	CX=4554H, SI=524BH, DI=4D41H

CY ==> No TEKRAM DOS5 support device

NC, CX=BAABH, SI=ADB4H, DI=B2BEH ==> TEKRAM DOS5 support device exist

AH=6	DL,80--87,CX=BAAB, SI=ADB4, DI=B2BE

CY ==> Device not controlled by DOS5 support controller

NC, BX=IOPORT,AL low nibble = SCSI-ID,AL high nibble=translation method

Procedures to determine AHA compatible device and TEKRAM DOS5 device

1. DL=80h, AH=6 ==> int13 ==> BX=ID of first SCSI device

DL=81h, AH=6 ==> int13 ==> BX=ID of first SCSI device

for all other drive #

DL=8X, AH=6, CX=4554, SI=524b, DI=4D41, INT13 to check if TRM BIOS device

if it is ==> INT13 again to get the IOPORT and ID, translation method

Programming information

;==========================================================

;==========================================================

; Mailbox interface

;==========================================================

;==========================================================

************************************

Adaptec compatible mailbox structure

************************************

CCB structure

byte 0	Command control block opcode

1	tar/init | data out | data in | LUN

2	SCSI command length ( = m )

3	Request sense allocation

4	Data length ( MSB,MID,LSB )

7	Data pointer ( MSB,MID,LSB )

10	Link pointer ( MSB,MID,LSB )

13	command link ID

14	Host status

15	Target status

16	Reserved ( byte #3 for data pointer )

17	Reserved

18	SCSI command bytes ( m bytes )

18+m	Sense data buffer

For Adaptec compatible

byte 16,17 are reserved

Command control block opcodes are 00,01,02,03,04,81h

Addressing limitation is 16M bytes

;==========================================================

; Tekram enhanced OPCODE for breaking 16MB limitation :

;==========================================================

Opcode	10h	SCSI Initiator command control block

12h	SCSI Initiator command control block with

scatter gather

Place byte #3 ( The most significant byte ) of data pointer

at CCB offset 16. Address list format is also modified to break

16MB limitation when scatter gather is required.

*********************************************************

Modified address list format for scatter gather operation

*********************************************************

Maximum number of scatter gather list entries is 17

each entry have 4 bytes of data length, and 4 bytes of address

Address list :

Entry #0	data length byte #0, #1, #2, #3, address byte #0, #1, #2, #3

Entry #1	data length byte #0, #1, #2, #3, address byte #0, #1, #2, #3

..

..

..

Note : Address list is modified to LITTLE ENDIAN ( rather than BIG ENDIAN )

Mailbox initialization is not modified, so mailbox MUST be below 16MB

Data buffer and address list can be placed above 16M by using OPCODE

10h or 12h and place the MSB of the address at CCB offset 16, use

the modified address list format if scatter gather is desired.