Installing Trantor drivers for Windows NT

September, 1992

This NT driver release has been tested with the Developer's Release

Disc.  It should also work with later releases of Windows NT.

T13B.SYS        Driver for the Trantor T130 SCSI adapter

T128.SYS        Driver for the Trantor T128 and T228 SCSI adapters

T348.SYS        Driver for the Trantor T348 MiniSCSI Plus adapter

Currently, these are the only Trantor SCSI host adapters that are

supported under Windows NT.  There are no plans to support the older

T100 adapter or the T338 MiniSCSI at this time.

NOTE: The Trantor driver interfaces the Trantor SCSI adapter to the NT

system, it does NOT support SCSI devices directly!  Support for SCSI

devices is supplied by Microsoft as part of Windows NT.

Installation as a Secondary adapter

With this driver release it is assumed that you will be using the

Trantor SCSI adapter as a secondary adapter in your system.  In other

words, the hard drive you boot from will not be connected to a Trantor

SCSI card.

If you are installing a T128/T228 or a T130 SCSI adapter, make sure you

have it jumpered for INTERRUPT 5 (IRQ 5) for the installation... this is

the WINNT software driver default.  You can also select IRQ 7 or 3 after

installation if you like. The T348 MiniSCSI Plus reserves IRQ 15 by

default.

Sample INI files for different IRQ settings are on this distribution

disk.  They must be installed in a "DEVICE" subdirectory under

\HKEY_LOCAL_MACHINE\SYSTEM\CurrentControlSet\services\ as explained in

the NT documentation if you want to change the default IRQ for the card.

Install the NT files manually as described in the NT documentation under

Section 2.2 Alternate Installation - DOS2NT.  Copy your Trantor NT

driver into the \WINNT\SYSTEM\DRIVERS directory.

Once the system is running, run the REGEDIT program and add the

appropriate Trantor Txxx.SYS file to the Registry.  The name in the

Registry MUST match the name of the driver.  Restart your system with

the Trantor SCSI adapter installed and the SCSI device connected.

Hard Disk Drives

Once the driver is installed in the Registry, and you have restarted

your system, you will be able to run Disk Manager and get an icon for

the SCSI drive.  You can format it for NT using the FORMAT command with

the /FS (File System) option you desire.

CD-ROM Drives

The Trantor driver supports the Trantor SCSI adapter only.  Drivers for

CD-ROM are supplied by Microsoft.  Only the CD-ROM drives supported by

Microsoft will be accessable, even using the Trantor driver.  Windows NT

should be able to recognize any supported CD-ROM drive when the system

is restarted.

Troubleshooting

If you are having trouble recognizing a SCSI device, make sure that you

can recognize that same device on the same computer running MS-DOS.  If

you don't recognize it under DOS, you won't recognize it under NT

either.

If all else fails, you can post a message on the WINNT Forum on

Compuserve (someone from Trantor will check there regularly), or leave a

message on the Trantor BBS at 510-656-5159.  Or you can FAX a question

to Trantor's technical support department at 510-770-9910.