STANDARD MICROSYSTEMS CORPORATION

Novell NetWare OS/2 Workstation Requester v2.0 Driver

For Ethernet LAN Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1993 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes:

o  How to use EZStart or a manual procedure to copy the Novell

NetWare OS/2 workstation requester version 2.0 driver from the

SuperDisk.

o  How to install the driver using the Netware OS/2 requester

INSTALL program.

o  How to install multiple adapters.

o  Error messages.

Note: This driver supports all 8216 (Ultra) and 80xx (PLUS) Series

ISA and Micro Channel Ethernet adapters.

COPYING THE DRIVER FILES

EZStart Method:

From the Driver Installation screen, select 'Copy Files' to copy

all files used by this driver from the SuperDisk to the Destination

Directory recommended by EZStart, or to a different directory that

you specify.

Manual Method:

The OS/2 Requester driver, SMC8000.SYS, and the NET.CFG file are

located in the \OS2_ODI subdirectory on the SuperDisk Driver

Disk. Run the Novell OS/2 Requester Install program, then copy

the SMC8000.SYS and NET.CFG files to the Destination Directory

specified in the Install program (the default Destination Directory

is \NETWARE).

LOADING THE DRIVER

Use the following instructions with those in the NetWare Requester

for OS/2 manual:

1. Load the NetWare INSTALL program according to the instructions

in the NetWare manual.

2. Select "Requester on workstation" from the menu.

3. Specify the source and target directories.

4. Select "Edit CONFIG.SYS and Copy Files".

5. Select "SMC8000.SYS" from the list. If the SMC driver is not in

the list, type: SMC8000.SYS

6. When prompted as to whether this is a third-party NIC driver,

select "Yes".

7. Select or deselect any remaining options, then select "Save".

In the "Save File As:" field, type: CONFIG.SYS

8.  In the Copy Requester Files window, select "Copy".

9.  Proceed with the instructions in the NetWare Workstation for

OS/2 manual until you complete the installation.

INSTALLING MULTIPLE ADAPTERS

If two adapters are installed in the same workstation, modify the

NET.CFG file in the \NETWARE subdirectory on your PC. To modify the

file, use any text editor to open the file and add the following

lines:

LINK DRIVER SMC8000

PORT XXX (ISA adapters)

SLOT X   (Micro Channel adapters only)

LINK DRIVER SMC8000

PORT XXX (ISA adapters)

SLOT X   (Micro Channel adapters only)

In the above lines, XXX is the I/O address of the adapter, in

hexadecimal. The first adapter should be the one with the lowest

I/O base address (use EZStart to obtain each adapter's I/O base

address).

After you modify the file, copy it to the \SMCNOV subdirectory or to

the directory where the SMC OS2 ODI driver resides.

ERROR MESSAGES

"Adapter hardware failure detected (initialization)"

The adapter did not pass the initialization process.

Check the adapter for configuration conflicts.

"Adapter hardware failure detected (self-test)"

The adapter did not pass the initialization self-test process

due to a hardware problem. Check the adapter for configuration

conflicts.

"Adapter IRQ higher than 7 not valid in 8-bit slot"

The I/O base address specified cannot be used if the adapter

is inserted into an 8-bit slot. Select a different IRQ number.

"No Ethernet Adapters Found"

The driver could not detect an adapter installed in the system.

Check the adapter is inserted properly into the slot connector.

"Invalid Ethernet node address specified"

The node address specified in the NET.CFG file was illegal.

Broadcast and multicast addresses are not allowed as valid

node addresses. Use the node address override with caution,

as this can cause problems on your network.

"One or more NET.CFG parameters are missing or invalid"

A driver parameter required to load the driver is missing from

the NET.CFG file. Add the parameter(s) and reload the driver.

"I/O address in NET.CFG is on illegal boundary"

The specified PORT address in the NET.CFG file is not correct.

Verify the adapter I/O address from the soft-configuration or

jumper setting, and make the necessary changes to the NET.CFG

file.

"I/O address in NET.CFG is out of range"

The I/O base address specified in the NET.CFG file for the

AT-bus adapter is out of range of supported values. Verify

the adapter I/O address from the soft-configuration or jumper

setting, and make the necessary changes to the NET.CFG file.

"IRQ in NET.CFG does not match adapter configuration"

The IRQ number specified in the NET.CFG file for the

AT-bus adapter does not match the detected configuration.

Verify the adapter IRQ setting from the soft-configuration or

jumper setting, and make the necessary changes to the NET.CFG

file.

"IRQ specified in NET.CFG is out of range"

The IRQ number specified in the NET.CFG file for the AT-bus

adapter is out of range of supported values. Verify the adapter

IRQ setting from the soft-configuration or jumper setting, and

make the necessary changes to the NET.CFG file.

"RAM address in NET.CFG is on illegal boundary"

The specified MEM address in the NET.CFG file is not correct.

Verify the adapter RAM memory address from the soft-configuration

or jumper setting, and make the necessary changes to the NET.CFG

file.

"RAM address in NET.CFG is out of range"

The RAM address specified in the NET.CFG file for the AT-bus

adapter is out of range of supported values. Verify the adapter

RAM memory address from the soft-configuration or jumper setting,

and make the necessary changes to the NET.CFG file.

"RAM size specified in NET.CFG is out of range"

The RAM size specified in the NET.CFG file for the AT-bus

adapter is out of range of supported values. Verify the adapter

RAM memory address from the soft-configuration or jumper

setting, and make the necessary changes to the NET.CFG file.

"RAM size in NET.CFG does not match adapter config"

The MEM length parameter in the NET.CFG file was invalid.

Check the adapter configuration, and make the necessary changes

to the NET.CFG file.

"Cannot access adapter RAM properly"

The adapter could read or write the adapter RAM. Verify that

another adapter does not share the same memory space as the

LAN adapter.

EtherCard Ultra, EtherCard PLUS, and SuperDisk are trademarks, and

SMC and Standard Microsystems are registered trademarks, of

Standard Microsystems Corporation. Other trademarks mentioned

herein belong to their respective companies.