IBM-PC MS-DOS Kermit V2.29 Terminal Emulator Summary

Keyboard Layout and Escape Sequences

Note that spaces shown between characters of escape sequences are there

for ease of reading; the actual sequences contain no spaces.

Part 1. The keyboard layout and characters sent by the terminal emulator.

Heath-19 and VT52 Keypads                     VT102 keypad

IBM Keys                                 IBM keys

+------+------+-------+----------+       +------+------+------+------+

| Blue |  Red |  Grey | up arrow |       |  PF1 |  PF2 |  PF3 |  PF4 |

|  F1  |  F2  |  F3   | up arrow |       |  F1  |  F2  |  F3  |  F4  |

+------+------+-------+----------+       +------+------+------+------+

|  7   |  8   |  9    |down arrow|       |  7   |  8   |  9   |  -   |

|  F5  |  F6  |  F7   |down arrow|       |  F5  |  F6  |  F7  |  F8  |

+------+------+-------+----------+       +------+------+------+------+

|  4   |  5   |  6    | rgt arrow|       |  4   |  5   |  6   |  ,   |

|  F9  |  F10 |  SF1  | rgt arrow|       |  F9  |  F10 |  SF1 |  SF2 |

+------+------+-------+----------+       +------+------+------+------+

|  1   |  2   |  3    |left arrow|       |  1   |  2   |  3   |  E   |

|  SF3 |  SF4 |  SF5  |left arrow|       |  SF3 |  SF4 |  SF5 |  n  S|

+-------------+-------+----------+       +-------------+------+  t  F|

|  0------0   |  .    |  Enter   |       |  0------0   |  .   |  e  6|

|  SF7        |  SF8  |  SF6     |       |   SF7       |  SF8 |  r   |

+-------------+-------+----------+       +---------------------------+

SF1 means push Shift and F1 keys simultaneously.

Cursor Keys:

H-19 & VT52               VT102

DEC/H-19 key    IBM key         all modes           Numeric     Application

------------    ---------       ---------           -------     -----------

up arrow        up arrow        ESC A               ESC [ A     ESC O A

down arrow      down arrow      ESC B               ESC [ B     ESC O B

right arrow     right arrow     ESC C               ESC [ C     ESC O C

left arrow      left arrow      ESC D               ESC [ D     ESC O D

Auxillary Keypad:

Heath-19 & VT52             VT102

DEC/H19 key     IBM key         Numeric Application    Numeric  Application

------------    -------         ------  -----------    -------  -----------

PF1/HF7/Blue    F1              ESC P   ESC P          ESC O P  ESC O P

PF2/HF8/Red     F2              ESC Q   ESC Q          ESC O Q  ESC O Q

PF3/HF9/Grey    F3              ESC R   ESC R          ESC O R  ESC O R

PF4/HF1         F4              ESC S   ESC S          ESC O S  ESC O S

0               SF7                     ESC ? p                 ESC O p

1               SF3                     ESC ? q                 ESC O q

2               SF4                     ESC ? r                 ESC O r

3               SF5                     ESC ? s                 ESC O s

4               F9                      ESC ? t                 ESC O t

5               F10                     ESC ? u                 ESC O u

6               SF1                     ESC ? v                 ESC O v

7               F5                      ESC ? w                 ESC O w

8               F6                      ESC ? x                 ESC O x

9               F7                      ESC ? y                 ESC O y

- minus         F8                      ESC ? m                 ESC O m

, comma         SF2                     ESC ? l (ell)           ESC O l (ell)

. period        SF8                     ESC ? n                 ESC O n

Enter           SF6                     ESC ? M                 ESC O M

(SFn means hold down Shift key while pressing Function key.)

Other IBM keys operational in Connect mode:

Del (White key) Send ascii Del code (rubout).

BackSpace       Send ascii Del code (rubout).

Shift BackSpace   Send ascii BS code (backspace).

Keypad (Grey)+  Toggle mode line on/off (only if Mode Line is enabled).

Alt -           Toggle among Heath-19, VT52, and VT100, but do not

change most operating parameters of the emulator.

Alt =           Reset terminal emulator to starting (setup) state.

Home            Roll screen up (text down) to beginning of storage.

End             Roll screen down to end of storage.

PgUp            Roll screen up (back, earlier) one screen full.

PgDn            Roll screen down (forward, later) one screen full.

Control PgUp    Roll screen up one line.

Control PdDn    Roll screen down one line.

Control PrtSc   Toggle on/off copying of received text to printer,

"PRN" shows on far right of mode line when activated.

Control End     Dump image of screen to a disk file or device.

Default filename is KERMIT.SCN in the current direct-

ory. Use command "Set Dump filespec"  to change the

filename or just "Set Dump"  to restore the default.

Screen images are appended to the file.

Shift PrtSc     Standard Print-screen, dump screen image to printer.

Kermit escape character ( ^] ) then one of the keys below:

?       display this short list.

F       dump screen to filespec, default is Kermit.scn.

C       close connect session & return to Kermit prompt.

S       show status of Kermit.

M       toggle mode line on/off.

B       send a Break signal.

P       push to DOS.

Q       quit (suspend) logging.

R       resume logging.

0       send a null character.

Kermit escape char      send it to the host.

-----------------------------------------------------------------------------

Part 2. Responses to characters received by the terminal emulator.

Unknown escape sequences of the form "ESC char" are absorbed by the emulator

without further effect.

DEC VT102 functions while in ANSI (VT102) mode

Escape seq      Mnemonic        Description of action

----------      --------        -------------------------------------------

ESC D           IND             Index, moves cursor down one line, can scroll

ESC E           NEL             Move cursor to start of line below, can scroll

ESC H           HTS             Set one horizontal tab at current position

ESC M           RI              Reverse Index, cursor up one line, can scroll

ESC Z           DECID           Indentify terminal (response is ESC [ ? 6 c)

ESC c           RIS             Reset terminal to initial state

ESC =           DECKPAM         Enter keypad application mode

ESC >           DECKNPNM        Enter keypad numeric mode

ESC 7           DECSC           Save cursor position and attributes

ESC 8           DECRC           Restore cursor from previously saved position

ESC # 3         DECDHL          Double height and width line, top half

ESC # 4         DECDHL          Double height and width line, bottom half

ESC # 5         DECSWL          Single height and width line

ESC # 6         DECDWL          Double width single height line

ESC # 8         DECALN          Screen alignment diagnostic - not supported

ESC [ Pn A      CUU             Cursor up Pn lines, does not scroll

ESC [ Pn B      CUD             Cursor down Pn lines, does not scroll

ESC [ Pn C      CUF             Cursor forward, stays on same line

ESC [ Pn D      CUB             Cursor backward, stays on same line

ESC [ Pn; Pn H  CUP             Set cursor to row, column (same as HVP)

ESC [ Ps J      ED              Erase in display:

0 = cursor to end of screen, inclusive

1 = start of screen to cursor, inclusive

2 = entire screen, reset lines to single

width, cursor does not move.

ESC [ Ps K      EL              Erase in line:

0 = cursor to end of line, inclusive

1 = start of line to cursor, inclusive

2 = entire line

ESC [ Pn L      (VT102)         Insert Pn lines preceding current line.

ESC [ Pn M      (VT102)         Delete Pn lines from current downward, incl.

ESC [ Pn P      (VT102)         Delete Pn chars from cursor to left, incl.

ESC [ Pn; Pn R  CPR             Cursor report (row, column), sent by terminal

Example: home position yields ESC [ 1; 1 R

ESC [ Pn c      DA              Device attributes (reports ESC [ ? 6 c)

ESC [ Pn; Pn f  HVP             Set cursor to row, column (same as CUP)

ESC [ Ps g      TBC             Tabs clear, 0 = at this position, 3 = all

ESC [ 20 h      LNM             Set newline mode (lf --> cr/lf)

ESC [ 20 l      LNM             Reset newline mode (lf --> lf)

ESC [ ? Ps;...;Ps h  SM         Set mode, see table below

ESC [ ? Ps;...;Ps l  RM         Reset mode, see table below

Ps      mnemonic        mode            set             reset

--      --------        ----            ---             -----

0                       error (ignored)

1       DECCKM          cursor keys     application   cursor (numeric)

2       DECANM          ANSI/VT52       ANSI/VT102      VT52

3       DECCOLM         Columns         *132 col        80 col

4       DECSCLM         *Scrolling      smooth          jump

4       (VT102)         Insert/replace  insert chars    replace chars

5       DECSCNM         Screen          reverse video   normal

6       DECOM           Origin          relative        absolute

7       DECAWM          Autowrap        on              off

8       DECARM          *Autorepeat     on              off

9       DECINLM         *Interlace      on              off

18      (VT102)         Printer termination char - not supported

19      (VT102)         Printer extent - not supported

(* note: 132 col, scrolling, autorepeat, interlace are not supported;

but 132 column command causes screen to be cleared.)

(Origin relative means with respect to top scrolling margin.)

ESC [ Pn i      (VT102)         Printer controls - not supported

ESC [ ? Pn i    (VT102)         Printer controls - not supported

ESC [ Ps;...;Ps m  SGR          Select graphic rendition

0 = all attributes off (#'s 1, 4, 5, 7)

1 = bold, intensify foreground

4 = underscore (shown as reverse video on

IBM CGA color systems)

5 = blink

7 = reverse video

non-DEC extensions:     30-37 = foreground color = 30 + colors

40-47 = background color = 40 + colors

colors: 1 = red, 2 = green, 4 = blue

ESC [ Ps n      DSR             Device Status Report.

Response from VT100: 0 = ready, 3=malfunction.

Command to VT100: 5 = report status with DSR,

6 = report cursor position using CPR sequence.

ESC [ Ps;...;Ps q DECLL         Load "LEDs", Ps = 0 means clear LED # 1 - 4

Ps = 1,2,3,4 sets LED # 1,2,3,4

ESC [ Pn; Pn r  DECSTBM         Set top and bottom scrolling margins

ESC [ sol x     DECREQTPARM     Request terminal parameters, see table below

ESC [ sol; par; nbits; xspeed; rspeed; clkmul; flags x

DECREPTPARM     Reports terminal parameters

sol = 0  request; term can send unsolicited

reports - not supported

sol = 1, request; term reports only on request

sol = 2, this is a report (DECREPTPARM)

sol = 3, terminal reporting only on request

par = 1 none, 2 space, 3 mark, 4 odd, 5 even

nbits = 1 (8 bits/char), 2 (7 bits/char)

xspeed,rspeed = transmit & receive speed index

0,8,16,24,32,40,48,56,64,72,80,88,96,104,112,120 correspond to speeds of

50,75,110,134.5,150,200,300,600,1200,1800,2000,2400,3600,4800,9600,19200 baud

clkmul = 1 (clock rate multiplier is 16)

flags = 0-15 (Setup Block #5), always 0 here

ESC [ 2; Ps y   DECST           Confidence tests - not supported

SCS             Select character sets.

ESC ( A         SCS             G0 points to UK symbols

ESC ) A         SCS             G1 points to UK symbols

ESC ( B         SCS             G0 points to ASCII symbols

ESC ) B         SCS             G1 points to ASCII symbols

ESC ( 0         SCS             G0 points to special (line drawing) graphics

ESC ) 0         SCS             G1 points to special (line drawing) graphics

ESC ( 1         SCS             G0 points to alt char ROM - not supported

ESC ) 1         SCS             G1 points to alt char ROM - not supported

ESC ( 2         SCS             G0 points to alt graphics ROM - not supported

ESC ) 2         SCS             G1 points to alt grpahics ROM - not supported

^G              BELL            Sound VT102 style beep

^H              BS              Backspace, move cursor left one character

^I              HT              Horizontal tab, move cursor to next tabstop

^J              LF              Linefeed, move cursor down one line

^L              FF              Formfeed, treated as a line feed

^M              CR              Carriage return, move cursor to col 1

^N              SO              Invoke usage of G1 character set

^O              SI              Invoke usage of G0 character set

Other extensions:

ESC [ ? 6 h  ESC [ 25; Pc f     VT52/VT100 move cursor to 25th line.

ESC [ ? 6 h  ESC [ 25; Pc H     VT52/VT100 move cursor to 25th line.

(These will disable Kermit's own status line.)

ESC * char                      VT200 series graphics command, ignored.

DEC VT102 functions while in VT52 mode

Escape seq                      Description of action

----------                      -------------------------------------------

ESC A                           Cursor up

ESC B                           Cursor down

ESC C                           Cursor right

ESC D                           Cursor left

ESC F                           Enter graphics mode

ESC G                           Exit graphics mode

ESC H                           Cursor home

ESC I                           Reverse line feed

ESC J                           Erase to end of screen

ESC K                           Erase to end of line

ESC Y row column                Direct cursor address, offset from space

ESC Z                           Identify (response is ESC / Z)

ESC =                           Enter alternate keypad mode

ESC >                           Exit alternate keypad mode

ESC <                           Enter ANSI mode (changes to VT102)

Heath-19 functions while in non-ANSI mode

Escape seq      Mnemonic        Description of action

----------      --------        -------------------------------------------

ESC A           HCUU            Cursor Up

ESC B           HCUD            Cursor Down

ESC C           HCUF            Cursor Forward, can jump to next line

ESC D           HCUB            Cursor Backward, can jump to previous line

ESC E           HCD             Clear display

ESC F           HEGM            Enter Graphics mode

ESC G           HXGM            Exit Graphic mode

ESC H           HCUH            Cursor Home

ESC I           HRI             Reverse Index

ESC J           HEOP            Erase to end of page

ESC K           HEOL            Erase to end of line

ESC L           HIL             Insert line

ESC M           HDL             Delete line

ESC N           HDCH            Delete character

ESC O           HERM            Exit Insert Char mode

ESC Y row col   HDCA            Direct cursor addressing, offset from space

ESC Z           HID             Identify (responds ESC / K  which is a VT52)

ESC b           HBD             Erase Beginning of display

ESC j           HSCP            Save cursor position

ESC k           HRCP            Set cursor to saved position

ESC l           HEL             Erase entire line

ESC n           HCPR            Cursor Position Report request

ESC o           HEBL            Erase beginning of line

ESC p           HERV            Enter Reverse Video mode

ESC q           HXRV            Exit Reverse Video mode

ESC r Bn        HMBR            Modify baud rate - not supported

ESC t           HEKS            Enter Keypad shifted mode, not supported

ESC u           HXKS            Exit Keypad shifted mode, not supported

ESC v           HEWA            Wrap around at end of line

ESC w           HXWA            Discard at end of line

ESC x Ps        HSM             Set Mode. See table below

ESC y Ps        HRM             Reset Mode. See table below

Ps      mnemonic        mode            set             reset

--      --------        ----            ---             -----

1       HSM/HRM         25th line       enabled         disabled

2                       *keyclick       off             on

3                       *holdscreen     enabled         disabled

4                       cursor type     block           underline

5                       *cursor on/off  on              off

6                       *keypad shifted shifted         unshifted

7                       alt app keypad  enabled         disabled

8                       *linefeed       cr = cr/lf      just cr

9                       newline mode    enabled         disabled

(* note: not implemented)

ESC z           HRAM            Reset to power-up configuration

ESC =           HAKM            Enter Alternate Keypad mode

ESC >           HXAM            Exit Alternate Keypad mode

ESC <           HEAM            Enter ANSI mode (ESC [ stuff)

ESC @           HEIM            Enter Insert Char mode

ESC [           HEHS            Enter Hold Screen mode, not supported

ESC \           HXHS            Exit Hold Screen mode, not supported

ESC { and }     HEK, HDK        Keyboard enable/disable, not supported

ESC ]           HX25            Transmit 25th line, not supported

ESC #           HXMP            Transmit page, not supported

Heath-19 functions while in ANSI mode

Escape seq      Mnenonic        Description of action

----------      --------        -------------------------------------------

ESC [ s         PSCP            Save cursor position & attributes

ESC [ u         PRCP            Restore cursor position & attributes

ESC [ z         PRAM            Reset to power-up configuration

ESC [ 2 J       ED              Erase entire screen but do not move cursor;

regular Heath-19 moves cursor to Home.

ESC [ ? 2 h     PEHM            Revert to normal Heath-19 non-ANSI mode

ESC [ > Ps h    SM              Same as ESC x Ps

ESC [ > Ps l    RM              Same as ESC y Ps

Plus most of the ANSI escape sequences listed for the VT102.

-----------------------------------------------------------------------------

Part 3. Miscellaneous notes:

o      Kermit requires from 60 to 98 K bytes of non-contiguous memory to run:

55 K bytes contiguous plus several other large blocks dynamically

allocated by DOS. Space is left for a second copy of Command.com.

o      Kermit captures only the Control-Break interrupt, Int 23H.

o      Kermit does not probe the keyboard typeahead buffer.

o      Kermit writes directly to screen memory but does so in a "TopView

Aware" manner to successfully run in windowing environments such as

TopView, Windows, and DESQview. It accepts text pasted from a window.

o      Kermit is aware of the IBM Monochrome, CGA, and EGA display adaptors.

o      Kermit expects the video mode to be TEXT, not graphics.

o      Kermit makes many Bios calls, principaly to the screen and keyboard.

o      All disk filenames are heavily checked; disk i/o uses only DOS calls.

o      Kermit directly accesses the serial port UART 8250 chip and that port

is interrupt driven for received characters. DTR and RTS are left

high when Kermit exits but otherwise the port state is restored.

o      The Hangup command will force DTR and RTS to be low (off).

o      Awarness of the IBM PC Network adaptor is present but deactivated.

Joe R. Doupnik

3 May 1986

[End of  MSYIBM.DOC]