1.  CREATE A DIRECTORY ON YOUR PALMTOP

To create a directory, go to the DOS prompt and type:

MD SSFAXER

Then type:

CD\SSFAXER to make sure you have created the directory.

(If you start with a .ZIP file you must unzip the file before porting the

files to the Palmtop.  You must have PKUNZIP.EXE.  If you don't have this file

you can download it from our BBS.  To unzip the file go the area or directory

where SSFAXHP7.ZIP is located.  Type the following:

PKUNZIP SSFAXHP7

Some users come on the BBS and download SSFAXHP7.ZIP and PKUNZIP.  Some

download using a desktop PC and then port the files to the HP after the file

has been unzipped.  Some download directly to the HP.  Those who download

using the HP can ignore step 2.

2.  PORT FILES FROM A PC TO THE HP PALMTOP

Use the Connectivity Pack or whatever means you have to port the SSFAXER files

on to your HP Palmtop.

Not all files are necessary.  If you want to SEND and RECEIVE faxes, load the

following files to the HP Palmtop:

SSSEND.ICN

SSRECR.ICN

SSSEND.EXE

SSRECR.EXE

SSFAXER.INI

COVER.DOC

If you do not plan to use a Cover Sheet, do not load COVER.DOC to the HP

Palmtop.

Another space saving feature - If you are not going to Receive faxes, you do

not need to load the following files:

SSRECR.ICN

SSRECR.EXE

SSRECER.EXE includes Receive, View and Print and can be loaded onto your HP

Palmtop at any time.

3.  SETTING UP THE ICONS

Now that you have all the necessary files loaded, you need to set the ICON.

The ICON file (ICN) must be the same name as the program it is attached to.

The ICON for SSSEND.EXE must be SSSEND.ICN and the ICON for SSRECR.EXE must be

SSRECR.ICN.

The ICON - ICN file must reside in the same directory as the program.  In this

case the ICN files must be in the A:\SSFAXER directory.

When you turn on your machine a variety of different screens can come up

depending on how you have your HP Palmtop setup.  Bring up the screen with the

ICONS.  Use the down arrow or PG DN key or TAB and go to the last line of the

ICONS.

Press F2 to ADD an ICON.  When you press F2 a screen comes up - fill in the

blanks.

Name:  The ICON Name - Type in SS Send.

Path:The File Path - Type in a:\ssfaxer\sssend.exe |360

This is the complete file path name and the " | " and the memory allocation.

This will give SSFAXER enough memory to be run in the Application Manager.

Comments:There is a problem using the Serial Port while the System Manager is

loaded.  This can    be eliminated by inserting an upside down exclamation mark

in the Comments area.  To   enter an upside down exclamation mark, press FN and

FILER.

Icon:  Use the up or down arrow and select an ICON and press ENTER.  If you

are satisfied    with the information you have entered, press F10.  If you

want to make changes to any    of the data entry fields, press the TAB key and

go the fields and edit them.  Then press    F10 to Save the information.

If you are not satisfied with the ICON, press F3 and edit it again.

Now you are ready to use SSFAXER.  TAB or down arrow to the SS SEND Icon.

Press ENTER.

Notice in the upper left corner you will see Send.  When you are in the

receive mode of SSFAXER you will see Receive in the upper left corner.

EXPLANATION OF THE MENU BAR

Files: Press Enter or F and then you will see a drop down menu with the

following:

Send file.

Press S for your selection or you may press <ENTER>

Send file.

If you press S for Send file, you will see the following information on the

screen:

Fax Filename: Enter the name of the file you want to fax.  For example, if you

want to fax a file that is    in your root directory, type the following:

C:\CONFIG.SYS

If you want to send a file that is in another directory, for example type:

C:\TEST\MYLETTER.DOC

Valid file formats are:  ASCII, PCX, DCX and PCL, and files that have already

been    converted by the Convert selection under Files on the Menu Bar.

Fax Phone: Enter the fax number to dial.  Remember to use the long distance

code and area code if    necessary.

Use .INI  Type in SSFAXER.INI

The INI file is an initialization file for use with SSFAXER.  The .INI file is

activated by    press the Space or ZOOM bar.  When active there will be an "X"

in the box to the left of   the field.  The .INI file allows you to control

some parameters as you are faxing.

Cancel  Exit without faxing.  If you press Cancel or F9, you will return to

the SSFAXER screen.

OK  Start faxing.  If you press OK for F10, you will begin faxing.  You will

see the stages of    sending the fax on the screen.  When the fax has been

sent press any key or the <ESC>    key to return to the menu bar.

Setup: TAB to Setup and press Enter or M and then you will see a drop down

menu with the following:

Modem

Modem Setup Screen

Port - Enter the Com Port number.

IRQ - Enter the IRQ number.  Standard IRQ's are COM1 and COM3 use IRQ 4,  COM2

and COM4 use IRQ 3.

P Baud - Valid settings are 19200, 9600 and 4800.  The P Baud (Port Baud)

tells SSFAXER the rate at which to talk to your serial port.  Most of the

time, you will set this to 18200, but it varies from modem to modem.

F Baud -  Valid settings are 9600 and 14400.  The F Baud (Fax Baud) setting is

the rate at which data is transmitted to the remote fax device.  Many

fax/modems are capable of transmitting at 14400 baud.

Answer - Enter the number of times you want the phone to ring before it

receives a fax.

Class - Enter the modem class number.  SSFAXER is compatible with Class 1 and

Class 2.

Modem Init - Some fax/modems require a special modem initialization string, in

addition to the default string  that SSFAXER uses.  If your modem requires a

special initialization string, you can define that in this field.  Please

refer to your fax/modem manual for the best init string for your particular

modem.

Dial Time - Enter the maximum number of seconds that SSFAXER will wait for a

carrier from the remote fax device.  This should be set to at least 45

seconds, which should be adequate for most situations.

Dial Prefix - Enter the number or numbers that need to be prepended to a phone

number.  For instance, if you need to dial 9 to get an outside line on your

phone system, you need to set this option to 9.  On some systems, after

entering 9, you need to wait for a few seconds to get a dial tone, if this is

happening, add some commas after the 9.  Each comma will give about 3 or 4

seconds of addition time.

Pulse Dial - If you do not use a Touch Tone phone system, press the Space Bar

or ZOOM.  This will place an 'X' in the box at the left.  When there is an 'X'

in the box, the selection is active.  Otherwise, SSFAXER will assume you have

a Touch Tone phone system.

Error Correction - Some fax/modems are capable of automatic Error Correction.

If your fax/modem is such a device, you may want to activate the Error

Correction option.  Press the Space Bar or ZOOM.  This will place an 'X' in

the box at the left.  When there is an 'X' in the box, the selection is

active.  Otherwise, SSFAXER will assume you do not want to use the Error

Correction option.

Cancel - Exit without faxing.  If you press Cancel or F9, you will return to

the SSFAXER screen.

OK - Start faxing.  If you press OK for F10, you will begin faxing.  You will

see the stages of sending the fax on the screen.  When the fax has been sent

press any key or the <ESC> key to return to the menu bar.