Documentation for HPPRN

-----------------------

HPPRN is a nifty little program written by Gilles Kohl that enables you to turn a $5000 computer into a very intelligent serial-to-parallel converter! Kidding aside, this utility will enable you to hook your Palmtop to your desktop computer via the normal serial connection and print documents DIRECTLY FROM THE PALMTOP APPLICATION TO THE PRINTER. No more need to transfer files to the desktop, load them into a desktop application, and print from there.

What You'll Need

----------------

To run HPPRN, you will need three files:

HPPRN.EXE, which is the utility that does the print redirection.

X00.EXE, a communications driver for the com port.

LISTEN.BAT, a batch file that calls HPPRN and X00 to perform their duties.

Running HPPRN

---------------

Here are the steps to run HPPRN:

1. Hook your desktop and Palmtop machines up via the standard serial cable.

2. Configure the com port on your palmtop to 9600,N,8,1,P using the MODE command if you will be printing from DOS or using the printer communications settings in Setup if you will just be printing from sysmgr. For more information, see the section on setting the palmtop's port speed below.

3. Place the three files listed above into a directory and make sure you have enough space on your drive to store the print files you will generate from the Palmtop.

4. Edit the LISTEN.BAT file to set the ports you want to use. You will need to enter a COM port setting for capturing data and a printer port setting for printing. BOTH SETTINGS ARE FROM THE PERSPECTIVE OF THE DESKTOP PC, NOT THE PALMTOP.  One little quirk: the COM port number is 0-based; that is, 0=COM1, 1=COM2, etc.  So, for example,  if you wanted to set up to capture the print information from COM2 on your desktop and reroute it to LPT1 Then you would edit LISTEN.BAT as follows:

set _cpth=1

set _ppth=lpt1:

The first parameter, _cpth, is the com port number (less 1) and the second parameter, _ppth, is the printer port. When you are finished, save the file.

5. Then just type in LISTEN and press the enter key. The LISTEN file will tell you what settings you are using and allow you to print directly from the Palmtop to the printer.

HOWEVER: There are a few restrictions and conditions:

You'll need Ray Gwinn's X00 FOSSIL driver or a compatible one. To find it, GO IBMFF and search for X00 or FOSSIL. The LISTEN.BAT file is configured to use X00.EXE, so if you want to use something else, you'll have to edit the file and figure out appropriate settings.

You'll have to read the X00 documentation and find out about the switches and parameters required for your system. I just used the lone parameter in the LISTEN.BAT file and found it was all I needed.

If you are using Windows, consider getting out of it and going to DOS (without Windows) before running LISTEN. I did not have good luck running from within Windows. (QEMM exceptions out the ying yang...) I am sure there is a way to configure a PIF file so that it would run, but I ran out of patience trying to get it to work. I had no problems outside of Windows.

This process works fine with network printers. Just use the printer as though it were locally attached. Note that if you are using Windows for Workgroups to get to your network devices, when you get out of Windows you will lose all of your connections. If you forget to set up a network printer from DOS and run LISTEN.BAT it will go on as though everything were just fine--except you will not get any output. All the print output just goes to the bit bucket.

I tried using HPPRN with and without LLRAS running on the desktop. It worked both ways and I found no conflicts.

LISTEN.BAT sets and clears two environment variables. If you are bumping up against the upper limit of your environment space on your desktop, just change all references to these variables to the settings you want to use and delete the statements that set and clear the variables. You get a little less flexibility, but you won't use the environment space. Another alternative is to expand your environment using whatever method is appropriate for the version of DOS you're running. Usually it's something like a /e parameter on the SHELL statement in CONFIG.SYS but it's different for different versions of DOS so check your manual.

Setting Your Palmtop's Port Speed

-----------------

There are two ways to configure your printer port speed on the HP. One is for printing from DOS applications and the other is for printing from sysmgr applications. Here are the details:

--If you are printing from a DOS application, use the MODE command. Enter the following from the DOS prompt:

MODE COM1:96,N,8,1,P

Make sure your DOS application is using the appropriate printer driver for your

printer. Go into your application and continue with the instructions

above.

--If you are printing from sysmgr applications, you need to set the port parameters in Setup. Go into Setup (Ctrl-Filer) and select Menu|Options|Printer from the menu. This brings up the printer options dialog box. Set the baud rate to 9600 and the interface to COM1. Then choose the printer driver you need, e.g. HP Laser Jet. Press F10 to save the settings, then quit out of Setup. You are ready to proceed with the rest of the instructions for printing.