User's Guide

BATTLOG (Battery Voltage Logger/Viewer)

Version 1.4 (superseding Version 1.2)

[1] What is BATTLOG?

BATTLOG is a combination of two programs, BATTLOG.COM and

BATTLOG.EXM.

BATTLOG.COM is a DOS TSR program, which monitors the voltage of

the two battery sets of the HP 200LX and records it in a log

file at 5 minute interval. (Measurement is done at 10 second

interval and the average, minimum and maximum of 30 readings -

5 minutes worth - are calculated and written to C:\BATTLOG.LOG)

BATTLOG.EXM is a system manager compliant program for viewing

of this log file in graphic presentation.

Altogether it is useful for checking the changes in battery

voltage over time and determining its end of life.

Since this is a TSR (Terminate and Stay Resident) program, the

same care should be exercised as in the cases of other TSR

programs.

This program was developed by MSC6 and MASM5 and tested by

the HP 200LX.  It works on the HP 100/200LX.

[2] Installation

1) Place BATTLOG.COM to the directory defined by the PATH statement

in your AUTOEXEC.BAT.

2) Insert a line

BATTLOG

in your AUTOEXEC.BAT.

3) Place BATTLOG.EXM and BATTLOG.ICN in your preferred directory,

4) In Application Manager ({More} Key), press {F2} (add), enter

necessary information to add BATTLOG.EXM and press {F10}

(OK).

5) Reboot by pressing Ctrl-Alt-Del.

Note: Generally it is not necessary to unload BATTLOG.COM

from the memory.  However, in case of need (if you do

not want to record the voltage), you can remove it by

exiting System Manager and executing BATTLOG again

from DOS prompt.

[3] How to use BATTLOG.EXM

BATTLOG.EXM read the log file C:\BATTLOG.LOG and makes a

graphical presentation on screen.  When the log file is

updated, it refreshes the screen.

It automatically creates an environmental file

C:\_DAT\BATTLOG.ENV.

1) Hitting {Menu} key will lead to the following menus.  (Values

marked by '*' is a default value)

{FILE}                   Log file operation.

{FILE} {Open...}        Open the log file for display.

{FILE} {Save As...}     Save the log file in a separate file.

{FILE} {Reset Logfile}  Delete the log file C:\BATTLOG.LOG.

(BE CAREFUL!!)

{FILE} {Exit}           Quit from the viewer

{View}                   Viewing options.

{View} {Volt Range>}    Setting voltage span. Select from

1.2V, 0.9V, *0.8V or 0.6V

{View} {Maximum Volt>}  Setting maximum voltage.  3.3V,

3.2V, *3.1V and 3.0V are available.

{View} {Time Range>}    Setting time range for one screen.

Choice of 48Hr, *24Hr, 12Hr or 6Hr.

{View} {Plot Mode>}     Selecting plotting mode.

Average (displays average values)

MaxMin  (displays maximum and minimum)

Both    (displays Maximum, Average and

Minimum)

{View} {Guide Line}     Toggles showing the horizontal grid

on the screen.

{View} {Power On/Off Guide}  Shows power cycles by vertical

bar.  Interval grater than 5 minutes

is marked by this line.

{Reference}              Display another graph on the same

screen.

{Reference} {Open...}   Open a reference file.

{Reference} {Close}     Close the reference file.

{Reference} {Graph}     Toggles showing the reference graph

on the screen.

{Options}                Measurement option.

{Options}  {Measure Interval>}  Choice of *5Min, 1 Min and

Auto Detect (Should be set at 5 min)

{Options}  {BattLog Trigger}   This item should not be

checked, since it does not work in

this version.

{Quit}                   Quit from the program.

{Help}                   (Help is not available in this

Version)

{Help}{About...}       Shows the version information

2) Scroll keys

To horizontally scroll the graph, you can use following

keys:

{Left Arrow}    Scroll left

{Right Arrow}   Scroll right

{Home}          Scroll to the beginning of the graph

{End}           Scroll to the end of the graph

3) Following Function keys are available mainly as a shortcut

keys.

{F1} (Help)    Help is not available in this version.

{F2} (Ave)     Same as {Menu}{View}{Plot Mode>}{Average}

{F3} (MaxMin)  Same as {Menu}{View}{Plot Mode>}{MaxMin}

{F4} (Both)    Same as {Menu}{View}{Plot Mode>}{Both}

{F5} (Home)    Same as {Home}

{F6} (<=)      Same as {Left Arrow}

{F7} (=>)      Same as {Right Arrow}

{F8} (End)     Same as {End}

{F9} (Open)    Same as {FILE} {Open...}

{F10} (Info)   Display current statistics

4) Indicators

Charging of Ni-Cd battery is indicated by a bar at the

bottom of the graph area.  Thick bar shows high current

charging in progress (first 6 hours) and thin line is

for low current (afterward).

Communication port status is indicated by a bar above battery

-charging bar.  Thick bar shows that Serial (RS232) port is

ON.  Thin bar is for IR port.  (Note: Status of PCMCIA card

modem is NOT indicated by this bar)

'!' sign at the upper right corner shows that BATTLOG.COM

TSR is loaded in place.  (If you are using other TSR

programs, this indicator may not correctly detect

BATTLOG.COM.)

'A' or 'B' sign at the lower right corner shows the system is

operating by AC Adapter (A) or by battery power (B).  'C'

is shown while N-Cd charging is in progress.

[4] Warning about the log file

���������������������������������������������������������Ŀ

�   Warning: The log file grows in size without notice!   �

�����������������������������������������������������������

BATTLOG.COM is always appending data to the logfile

C:\BATTLOG.LOG, which naturally tends to become larger

and larger.  Growth rate is 20 bytes per 5 minutes,

approximately 2 KB per 8 hours.

Notes:

- You can delete C:\BATTLOG.LOG any time.

- BATTLOG does not work while the power is OFF.  No data are

written unless the power is ON.

- Battlog appends the data until the disk is full.

[5] Usage Tips

I formerly used NiMH rechargeable batteries.  Now I switched to

alkaline.  My practice is as follows:

o When "Battery Low" appears on upper left screen, I prepare

for changing the battery.

o Before changing, I check with BATTLOG.EXM and save the log

file by Menu - File - Save As...  My naming convention for

this file is ALKAyym.LOG, where yy is the last 2 digit of the

year and m is month (1,2,3 to 9,A,B,C.  A to C is October to

December).  If it is second file in a month, I name it as

ALKA9592.LOG (2nd file of September 1995) for example.

o After saving the file, I delete the log file by Menu - File

- Reset Logfile.

o "Very Low" appears on screen, if you continue to work with

"Battery Low" status, dangerous for a flash disk user.  Writing

to a flash disk under this condition sometimes destroy the file

system on the disk.  I make it my own rule to change the

battery within one hour after the first "Low" warning.

o Performance of a battery varies much by its suppliers.  Some

reaches "Low" warning in a very short time.  The instruction

manual of my DoubleFlash Plus 20M (ACE) recommends not to use

TOSHIBA nor DURACELL.  Panasonic Gold seems to have a longer

life for me.

o Battery life is apparently longer, if I avoid using flash

disk as much as possible.  Same is true for the usage of

serial port.  Watch out for the bar (port is ON!) in the

graph shown by BATTLOG.EXM.

[6] Copyright

This program is a free software.  Its copyright is owned by its

author GAOGAO (S.Ieiri).  I plan to make it a shareware, since

I have got married <G>.

[7] Disclaimer

The author is not responsible for any damages which may be

caused by use or non-use of this program.

[8] Acknowledgement

Improvement from v1.2 to v1.4 owes much to the ideas posted by

the participants of "The road to the masters" Section of the

FHPPC Forum.  I would like to thank all of you who wrote your

valuable opinions there.

I am looking forward to hear from you again in the same

location.

10/27/1995

tcc02465@niftyserve.or.jp

GAOGAO(S.Ieiri)

Appendix A: For those updating from Ver 1.2

1. New features of Ver 1.4:

o Log file format is changed in V1.4, which is not compatible

with V1.2

o Files names of the executables are changed as follows:

BATTLOG5.COM to BATTLOG.COM

BATTLOGV.EXM to BATTLOG.EXM

o Recording of ON/OFF cycles of serial COM port is implemented.

o Viewer BATTLOG.EXM is now LHAPI compliant.

2. Procedures

1) Terminate System Manager and return to DOS prompt.  Then

release BATTLOG.COM (or BATTLOG5.COM) from memory.

2) Delete C:\BATTLOG.LOG (which is not compatible with new

version).

3) Start new BATTLOG.COM by modifying batch file to start

it.

4) Start System Manager.

5) On Application Manager, change the registered name from

BATTLOGV.EXM to BATTLOG.EXM.

3. Remarks

o V 1.4 has standard measurement interval of 5 minutes.  If you

are using V 1.2 under the name of BATTLOG5.COM, you should

change it to BATTLOG.COM.

o When you are updating to a new file, please make sure that

BATTLOG.COM (TSR) has been unloaded.

o Viewer BATTLOGV.EXM changes its name to BATTLOG.EXM.  Your

should change registration entries of AppManager.

o IMPORTANT NOTICE: Current C:\BATTLOG.LOG should be deleted

before installation of V.1.4, which has different output data

format.

Appendix B: Output File format

Output data are 20 byte binary data consisted of:

o Measurement date and time

o Average voltage of main battery

o Average voltage of backup battery

o Maximum and minimum voltage of main and backup batteries.

o active time of communication port

The format is as follows:

typedef struct tagDAT {   //20 byt/struct

char  date[4];      //BCD format, Century-Year-month-day)

char  time[4];      //BCD format, hour-minute-second-\0)

UINT  mainVolt;     //Average (main) e.g. 3000->3.000V

UINT  subVolt;      //average (backup)

UCHAR mainVoltMax;  //Max & Min (main)

UCHAR mainVoltMin;

UCHAR subVoltMax;   //Max & Min (backup)

UCHAR subVoltMin;

int   status;       //Status (BattLow or others)

UCHAR serial;       //Count of Comm port usage

char  reserved;

} DAT;

(UINT  = unsigned int)

(UCHAR = unsigned char)

Notes:

o Average voltage is an average of 30 readings sampled at 10

second interval.

o Battlog stops working when the power is off.  When the power

is on, it continues the interrupted measurement.

o Data file C:\BATTLOG.LOG can be deleted any time.

o Measurement interval may be prolonged while LapLink is

working.

[  Translated from Japanese by:  ]

[  Toshiki Sasabe (Janbe)        ]

[  73560.524@compuserve.com      ]