File Maven (tm) 3.5

Technical/Misc Information

System Requirements

-------------------

File Maven will run on any 8088/286/386/486/Pentium compatible PC that has

640K of conventional RAM (460K free) and DOS 3.0 or higher.  File transfers

work best when not run under Windows.

Serial Link Null Modem Cable

----------------------------

There are basically two types of null modem cables: 3-wire and 7-wire.  The

3-wire cable contains only transmit, receive, and ground lines.  The

7-wire cable has four additional handshaking lines.  File Maven supports

both cables but users should get a 7-wire cable if possible for faster file

transfers.  A 7-wire cable can be used in 3-wire link mode, but the reverse

is not true.  Null modem cables are available in most computer supply stores.

If possible, look for a 4-headed null-modem cable that can support both 9-pin

and 25-pin serial ports at either end.

DB25 DB9            DB9  DB25

---- ---            ---  ----

2    3   TX -- RX   2    3

3    2   RX -- TX   3    2

4    7  RTS -- CTS  8    5    <-- 7-wire only

5    8  CTS -- RTS  7    4    <-- 7-wire only

6    6  DSR -- DTR  4   20    <-- 7-wire only

7    5   SG -- SG   5    7

20    4  DTR -- DSR  6    6    <-- 7-wire only

3-wire/7-Wire Null Modem Pin Configuration

Parallel Link Cable

-------------------

There are basically two types of parallel link cables: 4-bit and 8-bit.

The 8-bit cable contains more lines and uses an additional register on the

parallel port for faster data transfer. File Maven supports both cables.

An 8-bit cable can be used in 4-bit mode but the reverse is not true.

The parallel link has three advantages over the serial link:

1. The 8-bit parallel link is up to 3 times faster than the 7-wire serial

link at the fastest baud rate.

2. All standard parallel ports are 25-pin, so 4-headed cables are never

needed.

3. There are no baud rate settings with the parallel link.

DB25M            DB25M

-----            -----

1 �������������  1     <-- 8-bit only

2 ������������� 15

3 ������������� 13

4 ������������� 12

5 ������������� 10

6 ������������� 11

10 �������������  5

11 �������������  6

12 �������������  4

13 �������������  3

14 ������������� 14     <-- 8-bit only

15 �������������  2

16 ������������� 16     <-- 8-bit only

17 ������������� 17     <-- 8-bit only

25 ������������� 25

4-bit/8-bit Parallel Pin Configuration

Cable Source

------------

A good place to buy cables is Computer Gate International at (408) 730-0673.

They carry both serial (p/n CNL0925FF) and parallel (p/n CNL2525MM) "Laplink"

style cables that work with File Maven.  See their ad in the Computer Shopper

or check out www.computergate.com.

Ports and Interrupts

--------------------

File Maven reads the serial and parallel ports addresses from a 14-byte

array in your computer's BIOS data segment starting at 0040:0000.  The four

possible COM port addresses are contained in the first 8 bytes and the three

possible LPT port addresses are contained in the next 6 bytes.  File Maven

uses direct polling instead of interrupt driven communications for the

fastest possible connections.  Therefore, there no are IRQ settings to

adjust.

Blowfish File Encryption

------------------------

File Maven uses Blowfish in ECB mode with a 32-bit key for file encryption.

Blowfish, a variable size key block-encryption algorithm, was developed by

Bruce Schneier, author of "Applied Cryptography" (Wiley), and presented in

the April 1994 edition of Dr. Dobb's Journal.

File Maven's implementation differs from the original specification in two

areas.  First, instead of initializing the s-boxes with the hexadecimal

digits of pi, File Maven uses the output of a pseudo-random number generator.

This eliminates the need for a separate 4k data file.  The PRNG equation

used is: rng(i)=rng(i-1)*134775813+1 where rng is a 32-bit variable

initialized to ffffffff hex.  Second, PCs with Intel CPUs store data in

little endian format (least significant byte first in a multi-byte word).

The original specification converts each block to big endian format before

encrypting and back to little endian format before saving.  Since this only

slows encryption and does not change the security of the algorithm, File

Maven skips the conversions.

File Maven uses the Secure Hash Algorithm (SHA) to generate the session

key.  The SHA was developed by The National Institute of Standards and

Technology (NIST), along with the National Security Agency (NSA).

File Maven takes the user's pass phrase, converts it to uppercase, and

runs it through the SHA.  This results in a 160-bit message digest, or

hash value.  Because of government restrictions on the export of

encryption techology, File Maven only uses the 32 most significant bits

for the session key and thus would only be considered to have "casual"

strength security.

Programming Language

--------------------

File Maven 3.5 was written using the Borland Pascal 7.01 compiler and

contains over 15,000 lines of code.  The low level Link functions were

written with in-line assembly code.  All code is the original work of

the author except for the compiler's run-time library.  No external

libraries were used.  Previous versions were marketed as shareware.

Contacting the Author

---------------------

For technical support or other inquiries, send a note to:

Kent Briggs

kbriggs@briggsoft.com

Visit the Briggs Softworks web site at www.briggsoft.com for information

on other shareware and freeware programs.