********************************************************************		         	                  CLEAN2.COM

********************************************************************

A simple program for IBM-PC to make full use of cleaning disks,

such as those sold by 3M and Verbatim.

Format of command:

X>CLEAN2 [drive:]

If no drive is specified, CLEAN2 assumes you want to work on the

default drive, else drive a:, b:, etc.	When it is through you

are asked if you want to clean any other drives, and if so

prompts for the drive letter.

This works on single or double sided or quad density drives

without modification and naturally will not accomplish too much

on your fixed disk. No harm will come, however, if it is run on

your fixed or floppy disk as it uses the PC-DOS absolute sector

READ to move the head around the disk.

The reason you use a cleaning utility rather than using DIR or

any other command to get the disk moving is that since the disk

in the drive is paper, the head will never get past Track 0 where

DOS goes for the directory in order to try to accomplish your

command.  In other words you will never use more than 2.5% of the

surface of your cleaning disk. CLEAN knows there is a cleaning

disk in the drive, ignores the DOS error message that the READ

has failed, and tries to read another sector on another track.

This way the head is moved about the disk and more effectively

cleaned.

Cleaning your disk drive is good preventive medicine, but don't

overdo it.  A drive that is in heavy use probably needs to be

cleaned once a week, but on the average once every other month is

plenty.

The reason for the 2 in CLEAN2.COM is that this is the second

version of this program.  At the time I wrote CLEAN I only had

single-sided drives and was not able to test CLEAN with

double-sided.  Thanks to a User Group Librarian I was set

straight on this and have improved CLEAN.  Users of the older

version may remember a patch I had provided for working with DSDD

drives.  This patch was not needed.  I am sorry for the

inconvenience caused by this and present this better working

version.

CLEAN now times in closer to the recommended 30 seconds for

cleaning a disk.  It also uses a random factor to start its

journey across the surface on different tracks.  Since it uses

only the most basic of MS-DOS functions, it should run on any

MS-DOS computer.

Program by:

Marty Smith  SOURCE ST2259 / COMPUSERVE 12155,1214

310 Cinnamon Oak Lane      (713) 661-1241 [Office]

Houston, TX  77079         (713) 464-6737   [Home]

[At my request, Marty has modified the program so that it will

work on any drive letter, so those of us with add-on quad disks

can be next to godliness, too.	Don't try sticking a cleaning

disk in your RAM drive, though!  R. Jueneman/SYSOP/(703) 237-4322]