Boca Research VGAX3 Video Drivers

This file contains 2 sections

1.  Installation Documentation for GEM 3.0

2.  Installation Documentation for Ventura Publisher

Section 1.

Installation Documentation for GEM 3.0

=================================================================

Overview

========

To install the GEM drivers the first step is to build a GEM

driver pak. The GEM DRIVER PAK is a floppy diskette that you create

that will contain the new SVGAX3 drivers. You will perform the following

step described below.

*   Format A blank diskette using the DOS FORMAT command. This

will be the new GEM DRIVER PAK

*   Copy 2 files from your GEM master diskettes to a temporary

directory on your hard disk (usually c:\temp)

*   Run the install program on the SVGAX3 diskette to install

SVGAX3 drivers to a temporary directory on your hard disk

(usually c:\temp)

*   copy the new SVGAX3 Drivers to your new GEM DRIVER PAK

*   run the GEMSETUP program

In Order to sucessfully complete the installation,

you should be farmilliar with the following DOS commands:

FORMAT

COPY

MKDIR

If you need information on these commands consult your DOS manual.

Installing the SVGAX3 drivers

=============================

|  Note: The following instructions use a temporary directory on your hard

|  disk 'C:\TEMP'. You may need to create this direcory by typing in the

|  following command:

|           MKDIR C:\TEMP

1.  Using the standard DOS FORMAT utility, format a blank

diskette by inserting it in drive A and typing the following:

FORMAT A: /V [Enter]

The DOS format utility will then prompt for the volume label by

displaying:

Volume Label? (11 characters max) Press Enter for none.

At this prompt, type the following volume-label name in upper

case:

GEM DRIVRPK [Enter]

2.  Using the DOS COPY utility, copy the following files from the

GEM/3 system master diskette (comes with GEM/3) to the temporary

directory on your hard-disk. Type the following sample

commands assuming that the GEM/3 system master diskette is in

drive A:

COPY A:GEMVDI.EXE c:\temp [Enter]

COPY A:MDGEM?.SYS c:\temp [Enter]

3.  Insert the SVGAX3 drivers diskette into Drive A:. Run the SVGAX3

installation program (INSTALL.EXE), and choose the option for the

GEM/3 drivers and specify the drive and directory where you want

them copied to.

If you choose to install the GEM/3 Drivers in the \TEMP subdirectory

of your hard disk (the default) type in the following command to copy the

new drivers to your new GEM DRIVRPK diskette

copy C:\temp\*.* A:

4.  After successfully creating the GEM DRIVER PAK, insert the GEM/3

System Master Disk in drive A.

Move to drive A by typing A:[Enter]. Please note that you must

install from the A: drive.

Then type GEMSETUP[Enter].

5.  Follow the GEM/3 installation instructions up to the CHANGE

EXISTING CONFIGURATION stage. When prompted for a screen driver

choice, highlight:

OTHER (DRIVER PACK)

When prompted to insert a screen driver into drive A, insert the

newly created GEM DRIVRPK diskette.

Select one of the following:

SVGAX3 16 Color 800x600  (if your monitor supports 800x600 resolution)

SVGAX3 16 Color 1024x768 (if your monitor supports 1024x600 resolution)

Continue with the remainder of the installation.

Section 2.

Instalation Documentation for Ventura Publisher

=================================================================

Installing Ventura Publisher display drivers

To install the Ventura Publisher driver, proceed as follows:

Install Ventura Publisher V2.0 as instructed in the product's

manual, specifying the Hercules driver.

Insert the SVGAX3 drivers diskette into Drive B:. Run the SVGAX3

installation program (INSTALL.EXE), and choose the option for the

Ventura Publisher drivers and specify the drive and directory

where you want them copied to (such as A:\). Ventura Publisher

requires that the drivers be installed from the root directory of

a floppy disk indrive A:. Be sure to install the drivers to the

root of drive A: or drive B: and then place the diskette in drive

A: before proceeding.

Change to drive A: by typing A:[Enter].

After the A: prompt appears, run the VPDRV2_0.EXE program by

typing VPDRV2_0[Enter]. Note that VPDRV2_0.EXE can only be

executed from the A: drive.

Follow the step-by-step installation procedure as it appears on

the screen and select one of the following drivers when prompted:

SVGAX3 800x600 16 of 256K colors or greys.

SVGAX3 1024x768 16 of 256K colors or greys.

Ventura Publisher V2.0 is now configured for the selected screen

resolution and 16 color graphics.