SCDMP.COM - Version 3.0

SCREEN DUMP UTILITY FOR H/Z-100

Introduction:

SCDMP  is a utility that allows reproduction of a complete video

screen  on  a  dot  matrix  printer,  including  both  text  and

graphics, without having to exit the current program.  The SCDMP

program  may  be  loaded  manually  (by  entering  SCDMP<cr>) or

automatically, via 'autoexec.bat', into memory at the  beginning

of a session where it remains resident until needed.  To print a

desired   stationary  screen,  simply  press  <SHFT-F12>,  which

generates an interrupt-5 and activates the screen dump.

The  program allows a choice of which color bank of video RAM is

dumped (if the user has color RAM in  his  Z100  and  the  COLOR

switch  is set to TRUE during assembly of the program).  For the

color version, entering a <B> for blue, <R> for red, or <G>  for

green immediately after the <SHFT-F12> will select the VRAM bank

to  be  printed.   If no character is entered, the green bank is

default.  If the COLOR switch is set to false,  only  the  green

bank can be dumped.

The  program  also  allows  multiple  density  printing for some

printers.  Entering an <H> immediately after the  <SHFT-F12>  or

color  selection would cause the printer to use a higher density

mode for printing.  Approximately two seconds is  allowed  after

initiation  of SCDMP before the default values are assumed.  The

default density is normal or standard density.

In  the  event  that  it  is necessary to stop the printing of a

screen, entering <ESC> during printing of the screen will  abort

the dump.

SCDMP can be activated from within many popular programs such as

Multiplan, ZBASIC, Graftalk, ZDS Business Graphics, Z-Chart, and

others.   SCDMP  will  not  work  with Lotus 123 or Watchword as

these programs disable the operation  of  the  interrupt-5.   It

conforms  to  the  operational  'standard'  created  by Zenith's

PSC.ASM, except that both graphics and text are reproduced.

Requirements:

This  program  requires the ZDOS/MSDOS operating system (Version

1.25 or higher) on a H/Z-100 computer.  The  printers  currently

supported  are  the C.  Itoh 8510A Prowriter with 2K buffer, NEC

8023A, Epson  MX/RX/FX  Series  with  Graftrax,  Star  Micronics

Gemini  Series, Okidata 80 Series, and Okidata Microline Series.

If MSDOS Version 2.13 is being used, the patch listed in  REMark

Volume  5,  Issue 12, December 1984, should be made to speed the

screen dump output to the printer.

Program Author:

Leslie L. Bordelon

1453 Fernwood Drive

Redlands, CA  92374

714-793-8853

Additional Credits to:

Digital Information Systems (Original Graphics Routines)

Zenith Data Systems (Interrupt-5 Handling from PSC.COM)

Distribution files:

SCDMP   .DOC	Screen Dump information

SCDMPPRO.COM	Color version for C. Itoh 8510A Prowriter

SCDMPPRO.ASM	  (supports three densities of printout)

SCDMPNEC.COM	Color version for NEC 8023A

SCDMPNEC.ASM	  (supports three densities of printout)

SCDMPEPS.COM	Color version for Epson MX Series

SCDMPEPS.ASM	  (supports two densities of printout)

SCDMPGEM.COM	Color version for Star Micronics Gemini Series

SCDMPGEM.ASM	  (supports two densities of printout)

SCDMPGMX.COM	Color version for Star Micronics Gemini 10X

SCDMPGMX.ASM	  (supports two densities of printout)

SCDMPOK1.COM	Color version for Okidata 80 Series (Step 1)

SCDMPOK1.ASM	  (single density only)

SCDMPOK2.COM	Color version for Okidata Microline Series

SCDMPOK2.ASM	  (supports two densities of printout)

SCDMP-RL.SKL	Skeleton versions for use in building a program

SCDMP-LR.SKL	  for your unique printer or application

SCDMP   .BAT	Batch file for auto load of SCDMP.COM

(rename the appropriate program to SCDMP.COM)

Program use:

Choose  the  version of the screen dump required by the specific

printer  application.   Rename  the  version  to  SCDMP.COM   as

follows:

REN SCDMPxxx.COM=SCDMP.COM

Load the screen dump program into resident memory as follows:

SCDMP<cr>

A  sign-on  message  will  display  the  version  number and the

printer application with instructions for  dumping  the  screen.

Screen dump instructions will be similar to the following:

To dump screen, type:  <SHIFT-F12>[X][Y]

where [X = <G>reen  or <B>lue or <R>ed bank]

and  [Y = <N>ormal or <H>igh density]

To abort dump during printing, type:   <ESC>

Typing  of  <SHIFT-F12>[X][Y]  will  suspend  execution  of  the

current program, dump the screen to the printer, and  return  to

the current program with no change.

Modifications:

The  source  code  is provided along with the assembled versions

for the above printers.  This is done to allow the user to adapt

the program to his specific printer or desires.   Comments  have

been  liberally  included in the source code.  Some knowledge of

assembly language programming is required, however.

If  a  monochrome  version  of the program is required, a simple

program modification is necessary.  This change will allow  only

the  green  bank of VRAM to be dumped to the printer.  The COLOR

equate located in the specific source listing for  the  selected

printer must be set equal to FALSE instead of TRUE.  The program

must then be reassembled as shown below under 'reassembly'.

There are two versions of the SCDMP program depending upon which

bits  of  the  graphics  byte the printer considers as the least

significant   and   most   significant   bits.    This   printer

characteristic  determines  which  direction to scan VRAM.  (For

example, the C Itoh requires a scan from right to left  and  the

Epson  requires  a  scan  from  left to right.)  Using the wrong

version results in characters being printed in reverse  (mirror)

image.   Another  approach  could  have been taken to solve this

problem, such  as  using  only  one  version  and  shifting  the

graphics  byte  to eliminate the reverse characters.  This would

have resulted in a slightly larger executable program.  Thus the

current approach of scanning the memory  table  in  reverse  was

chosen as the preferred method.

The  main  routines requiring change for adaption to a different

printer are as follows:

GO_PSC		Printer initialization routine

PRSPC		Spacing to center on page

MODE_G(K,L,S)	Printer graphics initialization

DO_COL		Graphics line termination

FINI		Printer reset routine

These are general areas  of consideration,  however the specific

routines depend upon your printer.

Two skeleton versions of the program are also provided as an aid

in building a program for a non-supported printer.  The skeleton

versions  present  an  outline  for both options of scanning the

Z100 screen depending on your specific type  of  printer.   With

this  data  and  a  little trial and error on the user's part, a

successful product for a unique printer can be developed.

Please  note,  if the unique printer is capable of only 7-bit or

less graphics, the routines used in SCDMPOK2.ASM will provide  a

good  example  to  use  in  modifying  the program.  The Okidata

Microline series printers display only 7-bit graphics,  the  8th

bit is ignored.  A bit shifting routine (SHFT_BIT) was added and

routines  DO_BYTE,  LNS,  and DO_COL were modified to insure all

eight bits were printed.

Reassembly:

To   reassemble   the  program  after  modification,  place  the

ZDOS/MSDOS system disk with MASM.COM, LINK.COM, and  EXE2BIN.COM

in  Drive  A:  and  the  data  disk with the modified version of

SCDMP.ASM in Drive B:.  Execute as follows:

MASM B:SCDMP,B:SCDMP;

LINK B:SCDMP,B:SCDMP;

EXE2BIN B:SCDMP.EXE B:SCDMP.COM

DEL B:SCDMP.EXE

DEL B:SCDMP.OBJ

Happy computing!